Skill Sheet 6-4

Objective 11	Cut an access opening in the skin of an aircraft (power saw). [NFPA® 1003, 4.4.1, 4.4.2]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will cut an access opening in the skin of an aircraft. The four-sided method of this skill requires students to work in groups of two or more: one to make the cuts, and one or more to catch the removed piece. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures. Firefighters must properly wear appropriate PPE when performing this skill.

Resources

| - Aircraft trainer or mockup with a cutaway panel - Charged handline - Pick-head axe, crash axe, or Halligan tool - Power saw (rotary saw) | - Air chisel - Padding/edge-protection material - Proximity Fire Fighting Protective Clothing, including SCBA or Structural Fire Fighting Protective Clothing, including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 6-4 Evaluation Checklist

Objective 11	Cut an access opening in the skin of an aircraft (power saw). [NFPA® 1003, 4.4.1, 4.4.2]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Three-sided Method |  |  |
| 1. | Select appropriate cut-in area. Some aircraft may have cut-in marks on the aircraft fuselage. |  |  |
| 2. | Create an access hole near the upper corner of the rectangle with either a pick-head axe or the point of a Halligan tool. |  |  |
| 3. | Insert the cutting blade into the hole and make a horizontal cut across the upper side of the rectangle. |  |  |
| 4. | Make vertical cuts from the ends of the horizontal cut down to the lower “cut-in” marks. |  |  |
| 5. | Fold the flap of aircraft skin downward toward the ground. |  |  |
| 6. | Pad jagged edges if time allows. |  |  |
|  | Four-sided Method |  |  |
|  | NOTE: This method must be used on composite materials. |  |  |
| 1. | Locate the “cut-in” marks on the aircraft fuselage. These marks outline a rectangle that is free of obstacles inside the fuselage or within the skin of the aircraft. |  |  |
| 2. | Create an access hole near the lower corner of the rectangle with either a pick-head axe or the point of a Halligan tool. |  |  |
| 3. | Insert the cutting blade into the hole and make a horizontal cut across the lower side of the rectangle. |  |  |
|  | NOTE: When completely cutting an access/egress panel, the bottom cut should be your first cut to avoid the saw blade or rescue tool from binding (caused by the weight of the door). |  |  |
| 4. | Make vertical cuts from the ends of the horizontal cut upward to the upper edge of the cut-in area. |  |  |
| 5. | Make a horizontal cut across the upper side of the rectangle. |  |  |
| 6. | Remove the rectangle of aircraft skin. |  |  |
| 7. | Pad jagged edges if time allows. |  |  |