Chapter 6

Hazard and Response Information

Lesson Goal

After completing this lesson, the student shall be able to describe the functions of a technical research specialist and identify the various kinds of references and resources that may be used in identifying hazardous materials and determining proper response.

Objectives

Upon successful completion of this lesson, the student shall be able to:

1.	Describe the job functions of a technical research specialist. [NFPA® 472, 7.3.3.4.5]

2.	Explain the basic principles of chemical research.

3.	Identify various written technical references. [NFPA® 472, 7.2.2.1(3)]

4.	Identify various electronic technical resources. [NFPA® 472, 7.2.2.1(1)]

5.	Describe other technical information centers and specialists. [NFPA® 472, 7.2.2.1(4, 5)]

6.	Determine characteristics of hazardous materials. [Learning Activity 6-1]

Instructor Information

This is the lesson covering hazard and response information. This lesson describes the functions of a technical research specialist. The lesson also covers written technical references and electronic technical resources. In addition, the lesson describes information centers and specialists that may be beneficial in determining hazardous materials response.

Important instructor information is provided in shaded boxes throughout the lesson plan. Carefully review the instructor information before presenting the lesson.

This lesson includes an activity that assists students in utilizing written and digital information regarding hazardous materials. The activity is developed for the student to gather information regarding a specific hazardous material and fill out the proper safety plan forms. This activity will cover learning objective 6.

Methodology

This lesson uses lecture, discussion, and a group learning activity. The level of learning is application.

Outline Contents

| Section | Outline Pages | Text Pages |
| --- | --- | --- |
| Section I: Introduction to the Technical Research Job Function | 5 | 226 |
| Section II: Research | 10 | 231 |
| Section III: Written Technical References | 17 | 235 |
| Section IV: Electronic Technical Resources | 36 | 244 |
| Section V: Technical Information Centers and Specialists | 42 | 248 |
| Section VII: Learning Activities | 48 | 252 |
| Section VII: Summary and Review | 48 | 251 |

Audiovisuals

- Visuals 6.1 to 6.57 (PowerPoint® Presentation)

Evaluation

- Chapter 6 Quiz

- Chapter 6 Test

- Learning Activity 6-1

Presentation Tools

Interactive Objects allow instructors to present information a piece at a time by clicking on hot spots in a larger image.

Each object is labeled in the Lesson Outline and indicated in the PowerPoint® presentation by a RED arrow in the top left corner of the image. This lets instructors know to use the mouse to explore all of the information on the slide.

After all of the information is displayed a reset arrow will appear in the bottom right corner of the image. Instructors can use this to remove the text and quiz students on content just covered or simply move on to the next slide.

To move to the slide after an interactive object, select the Click for next slide arrow on the left side of the slide.

Videos are used as discussion starters or to illustrate a concept or process discussed in the chapter.

These are labeled in the Lesson Outline and indicated in the PowerPoint® presentation by the Click image to play arrow on the left side of the slide.

Section I:	Introduction to the Technical Research Job Function

# introduction to the technical research job function

pp. 226-231	Objective 1 – Describe the job functions of a technical research specialist.

## Research Specialist Responsibilities

### Researching product information

### Predicting behavior of chemicals and containers

### Conducting appropriate communications and notifications

### Interpreting data for PPE selection

### Interpreting and analyzing technical information

### Completing documentation

## Risk-Based Response

### Uses information, science, and technology to mitigate hazardous materials incident

### Key – To equip responders with critical information that is needed to make good decisions

### Vital element – Product identification

### Hierarchy of decisions needed to protect responders; starts with strong, thorough size-up

### Quick and efficient way to “thin slice” information and make educated, lifesaving decisions

## Product Information and Behavior Prediction

### Allows haz mat technician to make well-informed decisions based on known chemical and physical properties of product or products involved

### Key components

#### Understanding physical properties of chemical

#### Understanding where information can be found

## U.S. Hazard Communication

### U.S. OSHA issues regulations relating to worker safety under Title 29 CFR

#### HAZWOPER regulation (29 CFR 1910.120)

#### Hazard Communication regulation (29 CFR 1910.1200)

#### Process Safety Management of Highly Hazardous Chemicals regulation (29 CFR 1910.119)

### Hazard Communication Standard (HCS)

#### Designed to ensure that information regarding chemical hazards and associated protective measures

#### Disseminated to workers and employers

#### Require chemical manufacturers and importers to

##### Evaluate hazards of chemicals they produce or import

##### Provide information about chemical through labels on shipped containers and safety data sheets (SDSs, formerly called material safety data sheets or MSDSs)

Note: SDSs are developed for industry and may not always be useful for emergency response.

## Data Interpretation for PPE Selection

### Chemical protective clothing (CPC) is subject to factors such as permeation, penetration, and degradation

#### Provided in product documentation

#### Documentation does not provide information on other hazards

WARNING! No Chemical Protective Clothing (CPC) can protect against all hazards.

### When determining appropriate PPE, answer the following questions

#### What is the product’s state of matter?

#### What is the product’s temperature?

#### Is it toxic by skin absorption?

#### Is it corrosive?

#### What is the vapor pressure?

#### What is the flash point?

#### Does the reactive properties?

#### Is it an oxidizer?

#### Is it radioactive?

### Questions can be answered by

#### Performing product monitoring

#### Researching chemical and physical properties of material,

#### Contacting technical specialist

Note: It is not always possible to determine exactly what the product is.

## Interpretation of Technical Information

### Wide range of reference materials exist

### Unique reference materials include

#### Chemical engineers

#### Industrial hygienists

#### United States Coast Guard

#### The Association of American Railroads

## Documentation of Research

### Critical at hazardous materials incidents

### Must be accurate

### Will be used for

#### Conducting hazard assessment concerning overall health of general public and emergency responders

#### Determining PPE and respiratory protection

#### Aiding decisions about chemical monitoring

### Must be organized

Review Question: What are the responsibilities of a technical research specialist at a haz mat incident?

See p. 227 of the manual for answers.

Section II:	Research

# research

pp. 231-235	Objective 2 – Explain the basic principles of chemical research.

## Research

### Has huge bearing on safety of incident

#### By understanding product’s physical and hazardous properties

#### Making informed decisions about protective actions

### Can help determine what effects product will have on humans and environment

### Allows responders to ensure that general public is protected

### Can show effect that product will have on chemical protective clothing

## Chemical Assessment – In order of priority, the following should be analyzed

Interactive slide; use mouse to show all information

Product identification

#### First priority

#### Elements assisting in locating product hazard information

##### Product name

##### Synonyms

##### DOT placard or label

##### UN/NA identification number

##### CAS number

##### NFPA® 704 marking

### State of matter – Due to dispersion characteristics has significant impact on personal protection and tactical operations

### Poison inhalation hazards – Due to toxicity and dispersion characteristics, present substantial life-safety threat; must be protected against accordingly

### Flammability hazards

#### Have significant effects on personal protection and tactical options

#### During research, obtain critical points

##### Flashpoint

##### Ignition temperature

##### Flammable or explosive limits

### Toxicity hazards

#### Require consideration of routes of entry and degree of toxicity, as well as physical characteristics affecting dispersion

#### Affect decisions about PPE and tactical options

#### During research, obtain critical points

##### Carcinogen

##### Teratogen

##### Mutagen

##### Route of entry

##### TWV-TWA

##### TLV-STEL

##### TLV-C

##### IDLH

##### Corrosivity/contact hazard

##### Ionizing radiation

### Dispersion characteristics

#### Method and pattern will affect exposure protection, selection of PPE, and control tactics

#### Key characteristics that should be researched

##### Vapor pressure

##### Boiling point

##### Vapor density

##### Specific gravity

##### Water solubility

### Reactivity characteristics

#### Substances may react with other materials present or those introduced by response personnel

#### Key characteristics that should be researched

##### Air reactivity

##### Water reactivity

##### Incompatibility

##### Polymerization

##### Temperature sensitivity

## Hazardous Material Profiling

### Efficient tool for size-up and hazard analysis

### Provides haz mat technician with organized approach to understanding physical and hazardous characteristics of product

### Allows for response that is well-informed and risk-based

### Identifies physical behaviors of product and associated hazards

### Should be performed using quality information that can be validated by monitoring and detection technology

### Essential that correct information be profiled

### Critical baseline information

#### State of matter – Physical form a chemical or product will be found in at normal temperatures and pressures

#### Polymerization potential – Chemical that has ability to polymerize will not be able to be controlled by initial response crews

#### Flashpoint – Volatile liquids will produce vapors that may ignite

#### Lower explosive limits (LEL) – Concerns about possible ignition sources must be addressed if product is flammable

#### Reactivity – Product may have flammable and corrosive by-products if it does react

#### Specific gravity – Weight of substance compared to weight of equal volume of water at given temperature

#### Vapor density – Weight of given volume of pure vapor or gas compared to weight of equal volume of dry air at same temperature and pressure

#### Vapor pressure

##### Pressure at which vapor is in equilibrium with liquid phase for given temperature

##### Liquids that have greater tendency to evaporate have higher vapor pressures for given temperature

#### Ionization potential – Energy required to completely remove electron from atom

### Should incorporate monitoring and detection equipment; must cover broad spectrum of response data

#### Radiation detectors

##### Any radiation reading above normal background levels are concern

##### Retreat immediately if elevated levels are detected

#### pH paper – Can be used to detect corrosive vapors in air, as well as to determine pH of those vapors

#### Reagent paper – Used to detect numerous substances

#### Combustible gas detectors (LEL meters)

##### Detect flammable atmospheres

##### May be stand-alone meters or contained within traditional four- or five-gas meter with other oxygen and toxicity sensors

#### Thermal detectors – Used to determine if chemical reaction or polymerization is taking place

### Ongoing process; as new information gathered, response objectives should be developed

CAUTION: Safety is always paramount in a haz mat response. Base decisions only on validated information. Incorrect or outdated information is not reliable.

Review Question: What are the two elements of chemical research?

See p. 231 of the manual for answers.

Section III:	Written Technical References

# written technical references

pp. 235-243	Objective 3 – Identify various written technical references.

## Written Technical References

### Provide haz mat technician with technical data on host chemical

### May suggest response options

### Provide information that includes

#### Chemical technical data

#### Physical and chemical properties of material

#### Treatment methods

#### Decontamination options

### MUST use most current edition of any reference

### Each begins with

#### Basic description of how manual is organized

#### Use of references and acronyms

#### Glossary of terms

## Reference Manuals

### Emergency Response Guidebook (ERG)

#### Primarily designed for use at transportation incidents involving hazardous materials

#### Intended to be starting point for research

#### Provides only general information on included chemicals

#### Provides

##### Quick guide to assist emergency responders in identifying chemical

##### Very general information on how to protect themselves and others while more specific information is acquired

##### Special information such as polymerization hazards

##### Cross-references based on UN/NA numbers or actual spelling of chemical

##### Primary identification means – Should only be used at onset of incident

#### Does not supply specific information – Can still be used to gather beneficial information on response

#### Quick reference to determine if material in transport is radioactive or corrosive

#### May be quickest way to determine if product will polymerize

#### Also establishes initial isolation distances based on involved product’s state of matter

##### Solids – 75 ft. (25 m)

##### Liquids – 150 ft. (50 m)

##### Gases – 330 ft. (100 m)

### Emergency Handling of Hazardous Materials in Surface Transportation

#### Developed by Association of American Railroads and Bureau of Explosives

#### First section

##### General information on approaching haz mat incident

##### Product identification

##### Response information for each DOT hazard class

#### Second section – Commodity-specific emergency response information for each hazardous material regulated by U.S. DOT, Transport Canada, and International Maritime Organization

#### Contains multiple cross-references

##### DOT UN/NA numbers and page references for many commonly shipped materials

##### Seven digit haz mat code referred to as standard transportation commodity code (STCC)

##### All seven digit haz mat codes, description of product assigned to code, and page number

### Chemical Hazards Response Information System (CHRIS)

#### Designed to provide information needed by U.S. Coast Guard personnel responsible for decision-making for incidents involving waterways

#### Volume 1

##### The Condensed Guide to Chemical Hazards

##### Good resource for first responders on scene of haz mat incident

##### Contains information specific to each hazardous material that may be encountered

##### Only useful if material has been properly identified

#### Volume 2

##### The Hazardous Substances Data Manual

##### One of most useful references when responding to water-based spills and waste sites

##### Contains information on hazardous materials commonly shipped in large volume by water

##### Used regularly by port security personnel

##### Easily understood

##### May be used as quick reference to determine needed actions to safeguard life and property

Note: The online CHRIS manual is no longer available; however, print copies are still widely used.

### Crop Protection Handbook

#### Formerly known as Farm Chemicals Handbook

#### Important reference to crop protection products, systems, inputs, regulations, safety, and sources

#### Contains detailed technical and safety information on thousands of fertilizer and pesticide products

#### Contains in-depth coverage of regulatory issues

### NIOSH Pocket Guide to Chemical Hazards (NPG)

#### Prepared by National Institute for Occupational Safety and Health (NIOSH)

#### Based on NIOSH and OSHA guidelines for chemical hazards

#### Based on NIOSH criteria documents and recognized references in field of industrial hygiene, occupational medicine, and toxicology

#### Presented in table format

#### Intended to provide quick information on general industrial hygiene practices

#### Includes

##### Chemical names, synonyms, trade names, CAS, RTECS, and DOT ID and Guide numbers

##### Chemical structure/formula, conversion factors

##### NIOSH Recommended Exposure Limits (RELs)

##### OSHA Permissible Exposure Limits (PELs)

##### NIOSH Immediately Dangerous to Life and Health values (IDLHs)

##### Physical description and chemical and physical properties of agents

##### Measurement methods

##### Personal protection and sanitation recommendations

##### Respirator selection recommendations

##### Incompatibilities and reactivities of agents

##### Exposure routes, symptoms, target organs, and first aid information

### NFPA® Fire Protection Guide to Hazardous Materials

#### Produced by National Fire Protection Association® (NFPA®)

#### Provides good information on properties of hazardous chemicals

#### Important for those who use chemical sin industry and those who respond to chemical emergencies including fires, accidental spills, and transportation accidents

#### Composed of six NFPA® documents

##### NFPA® 49, Hazardous Chemicals Data

###### Provides information on over 300 chemicals

###### Provides specific information on health, fire, and reactivity data

###### Arranged alphabetically by DOT shipping names

##### NFPA® 325, Guide to Fire Hazard Properties of Flammable Liquids, Gases, and Volatile Solids – Provides fire hazard properties for over 1,300 flammable substances

##### NFPA® 491, Hazardous Chemical Reactions – Includes information on over 3,500 mixtures of two or more chemicals that may be potentially dangerous at moderate or elevated temperatures

##### NFPA® 704, Standard System for the Identification of the Hazards of Materials for Emergency Response

###### Provides degrees of health, flammability, and reactivity for many of common industry chemicals

###### Newest edition also includes information on aerosol products

##### NFPA® 497, Recommended Practice for the Classification of Flammable Liquids, Gases, or Vapors and of Hazardous (Classified) Locations for Electrical Installations in Chemical Process Areas

###### Includes information on certain flammable gases and vapors, flammable liquids, and combustible liquids

###### For purposes of selecting appropriate electrical equipment in hazardous locations

##### NFPA® 499, Recommended Practice for the Classification of Combustible Dusts and of Hazardous (Classified) Locations for Electrical Installations in Chemical Process Areas

###### Provides information about certain combustible dusts

###### For purposes of selecting appropriate electrical equipment in hazardous locations

### Hawley’s Condensed Chemical Dictionary (CCD)

#### Compilation of technical information and descriptions that cover thousands of chemicals

#### Organized so that it offers three types of information

##### Technical descriptions of chemicals and processes

##### Expanded definitions of chemicals and terminology

##### Descriptions and/or identification of trade names

### Jane’s CBRN Response Handbook

#### Expanded version of Jane’s Chem-Bio Handbook

#### Field reference guide that allows responders to quickly identify chemical, biological, or radiological weapons and their effects

#### Contents include information about

##### Preincident planning

##### On-scene procedures

##### Postincident management

##### Chemical agents and treatment

##### Biological agents and treatment

##### Radiological hazards and treatment

### Merck Index

#### Publication intended to serve chemists, pharmacists, physicians, and other allied health professionals

#### Offers emergency responder vital information on chemicals, drugs, and biological information

#### Supplies concise information on thousands of chemicals and compounds

#### Records contain

##### Chemical, common, generic, and systematic names

##### Trademarks and associated companies

##### CAS registry numbers

##### Molecular formulas, weights, and percentage compositions

##### Capsule statements identifying compound classes and scientific significance

##### Chemical, biomedical, and patent literature references

##### Physical and toxicity data

##### Therapeutic and commercial uses

##### Caution and hazard information

### Sax’s Dangerous Properties of Industrial Materials

#### Multiple volume reference set

#### Provides hazard information for over 26,000 chemicals

#### Provides relevant information on

##### Toxicology

##### Reactivity

##### Explosive potential

##### Regulatory information

#### First volume – CAS number and synonym cross index

#### Remaining volumes contain

##### Dangerous Properties of Industrial Materials (DPIM) codes

##### Hazard ratings

##### Entry name

##### CAS numbers

##### DOT (UN/NA) numbers

##### Molecular formula

##### Line structure formula

##### Material descriptions

##### Toxicology data

### Symbol Seeker

#### Publication that helps first responders identify extensive number of international hazard markings

#### Provides means of identifying hazardous materials in any mode of transportation or on any fixed facility

#### Does not offer specific information on chemical, substance, or product

#### Intended for use in identification of material

#### Marking systems include

##### Haz mat marking systems

##### Nuclear and radioactive marking systems

##### Biohazard marking systems

##### Ordinance and weapon marking systems

##### Marking systems specific to individual countries

### The Threshold Limit Values and Biological Exposure Indices (TLVs® and BEIs®)

#### Published by American Conference of Governmental Industrial Hygienists (ACGIH®)

#### Designed for use by industrial hygienists and emergency responders

#### Provide occupational exposure guidelines for hundreds of chemical substances and physical agents

## Safety Data Sheets (SDSs)

### Detailed information bulletins prepared by manufacturer or importer of chemical

### Provide specific information about product

### Once called material safety data sheets (MSDSs)

#### Mandated by U.S. OSHA and Canadian authorities

#### Both countries converting to Globally Harmonized System (GHS) SDS format

#### May still be encountered

### Often good sources of detailed information about particular material

### Can be acquired from manufacturer, supplier, shipper, emergency response center, or facility hazard communication plan

### Sometimes attached to shipping papers and containers

### GHS for Hazard Classification and communication specifies minimum information

### Must include following sections

#### 1:	Identification

#### 2:	Hazard(s) identification

#### 3:	Composition/information on ingredients

#### 4:	First aid measures

#### 5:	Fire fighting measures

#### 6:	Accidental release measures

#### 7:	Handling and storage

#### 8:	Exposure controls/personal protection

#### 9:	Physical and chemical properties

#### 10:	Stability and reactivity

#### 11:	Toxicological information

#### 12:	Ecological information

#### 13:	Disposal considerations

#### 14:	Transport information

#### 15:	Regulatory information

#### 16:	Other information

## Shipping Papers

### Required to accompany a product in transit, regardless of means of transportation

### Can be reliable means of product identification

### In U.S.

#### Legally required by 49 CFR 172.604

#### Must include

##### Proper chemical name

##### Four digit hazard classification (UN/NA) number

##### Package types

##### Total quantity of product being shipped

##### 24-hour emergency contact number

#### HM-181 – Contains new requirements and scaled system

##### 1 for high hazard

##### 2 for medium hazard

##### 3 for low hazard

### Must include Standard Transportation Commodity Code (STCC)

#### If hazardous, preceded by number 49

#### If hazardous waste, preceded by number 48

## Facility Documents

### Chemical Inventory Lists (CILs)

#### Required of U.S. employers by Hazard Communication Standard (HCS)

#### Usually contain information about locations of materials within facility

#### Useful tools identifying containers that may have damaged or missing labels or markings

### Other useful documents or records

#### Shipping and receiving documents

#### Inventory records

#### Risk management and hazardous communication plans

#### Chemical inventory reports (known as Tier II reports)

### Local Emergency Planning Committee (LEPC)

#### Develop emergency response plans – Good source of information

#### Designed to provide forum for emergency management agencies, responders, industry, and public to work together to evaluate, understand, and communicate chemical hazards in community

#### Designed to develop appropriate emergency plans in case of accidental release of these chemicals

## Limitations

### Use most current edition of reference to ensure information is up to date

### Due to different testing methods, slight variations to chemical information may exist within written references

### Terminology may also vary between references

### Because of variations, use multiple sources to determine validity of information

### AHJ will dictate number and type of sources used

### Responders should use due diligence to ensure product is thoroughly researched

Review Question: What written technical references can be used to gather hazardous materials information?

See pp. 236-243 of the manual for answers.

Section IV:	Electronic Technical Resources

# electronic technical resources

pp. 244-248	Objective 4 – Identify various electronic technical resources.

## Electronic Technical Resources

### Most common written resources and references now available in electronic format

### Have added benefit of search features that allow information to be accessed in more efficient manner than print resources

## Chemical Databases

### Some available in software suite; others web-based

### Beneficial because do not require large areas for storage; can be easily updated

### Formatted differently – Review to decide which are most productive for organization

### Computer-Aided Management of Emergency Operations (CAMEO)

#### Designed by National Oceanic and Atmospheric Administration (NOAA)

#### System of software applications that assists emergency responders in development of safety response plans

#### Can be used to access, store, and evaluate information critical in emergency response

#### Database and information management tools

#### Extensive chemical database

#### Chemical datasheets

##### Physical properties

##### Health hazards

##### Air and water reactive information

##### Fire fighting information

#### UN/NA datasheet – Provides information from ERG and shipping information from Hazardous Materials Table

#### Mapping application

##### Mapping Applications for Response, Planning, and Local Operational Tasks (MARPLOT)

##### Allows users to visualize data and display information on computer maps

##### May be printed individually or on area maps

#### Dispersion modeling program

##### ALOHA

##### Used for evaluating release of hazardous chemical vapors

##### Allows user to estimate potential harm to area based on

###### Toxicological and physical characteristics of chemical

###### Atmospheric conditions

###### Any other specific circumstances that may be affecting release

### Wireless Information System for Emergency Responders (WISER)

#### Designed for use by first responders at hazardous materials incidents

#### Brings wide range of information to responder

#### Develops database from series of relevant sources

#### References chemical data consolidated from core set of reference manuals

##### Emergency Response Guidebook

##### Chemical Hazards Response Information System

##### NFPA® Fire Protection Guide to Hazardous Materials

##### NIOSH Pocket Guide to Chemical Hazards

##### EPA Integrated Risk Information System (IRIS)

##### POISINDEX® Information System

##### TOMES® Information System

##### ACGIH Guidelines for Selection of Chemical Protective Clothing

##### ACGIH® TLVs® and BEIs®

##### Emergency Handling of Hazardous Materials in Surface Transportation

#### Available in different formats depending on operating system

#### May be downloaded free of charge

## Weather Data

### Just as critical as chemical reference manuals

### Must be correctly interpreted

### Can have profound effect

#### Humidity

#### Atmospheric pressure

#### Temperature

#### Precipitation

#### Wind speed and direction

### More readily available than ever

### Portable weather stations

#### Allow for more exact weather information within Command Post

#### Many can provide

##### Barometric pressure

##### Temperature

##### Wind speed and direction

##### Relative humidity

## Plume Modeling

### Variety of programs available

### U.S. EPA – Visual Plumes

### Many provide user with ability to plot plume footprint on map

### Many can be used in conjunction with preexisting GIS mapping systems

### Limitations

#### Model must be matched with material

#### Forecasts any have single event errors

## Internet Resources

### Ansell Chemical Resistance Guide

### Agency for Toxic Substances and Disease Registry (ATSDR)

### NIST Chemistry WebBook

### ChemSpider

### CHEMnetBASE

### Pesticide Action Network (PAN) Pesticide Database

### Toxicology Data Network (TOXNET)

### Radiation Emergency Medical Management (REMM)

Note: Internet addresses change frequently and therefore are not provided in this text. A simple internet search of the listed databases will provide the most current information.

Review Question: What chemical databases may be used to gather hazardous materials information?

See pp. 244-246 of the manual for answers.

Section V:	Technical Information Centers and Specialists

# technical information centers and specialists

pp. 248-251	Objective 5 – Describe other technical information centers and specialists.

## Emergency Response Centers

### CHEMTREC®

#### Established by U.S. chemical industry

#### Public service hotline for emergency responders

#### Provides information and assistance for emergency incidents involving chemicals and hazardous materials

#### Staffed with experts who provide 24-hour assistance to emergency responders dealing with haz mat emergencies

### Canadian Transport Emergency Centre (CANUTEC)

#### Operated by Transport Canada

#### National, bilingual advisory center

#### Part of Transportation of Dangerous Goods Directorate

#### Has scientific data bank on chemicals manufactured, stored, and transported in Canada

#### Staffed by professional scientist who specialize in emergency response and are experienced in interpreting technical information and providing advice

### Mexican response centers

#### National Center for Communications of the Civil Protection Agency (CENACOM)

#### Emergency Transportation System for Chemical Industry (SETIQ)

#### Operated by National Association of Chemical Industries

### Emergency responders should provide center with as much information as possible

#### Caller’s name, callback telephone number, and FAX number

#### Location and nature of problem

#### Name and identification number of material(s) involved

#### Shipper/consignee/point of origin

#### Carrier name, railcar reporting marks, or truck number

#### Container type and size

#### Quantity of material transported/released

#### Local conditions

#### Injuries, exposures, current conditions involving spills, leaks, fires, explosions, and vapor clouds

#### Local emergency services that have been notified

### The emergency response center will do the following

#### Confirm that a chemical emergency exists

#### Record details electronically and in written form

#### Provide immediate technical assistance to caller

#### Contact shipper of material or other experts

#### Provide shipper/manufacturer with caller’s name and callback number so that shipper/manufacture can deal directly with party involved

## Poison Control Centers

### Provide free 24-hour advice to anyone located in fifty states, Puerto Rico, Micronesia, American Samoa, and Guam

### Provide poison expertise in treatment advice by phone

### Can be reached by calling 1-800-222-1222

## Chlorine Emergency Plan (CHLOREP)

### Developed by Chlorine Institute to assist in handling of chlorine emergencies in U.S. and Canada

### Operated through CHEMTREC®

#### Notifies nearest manufacturer in accordance with mutual aid plan

#### Manufacturer will contact someone within command structure to determine if response team is needed

## Interagency Radiological Assistance Plan (IRAP)

### Designed to assist when responding to radiological emergencies

### Operated under U.S. Federal Department of Energy

### Works closely with other federal, state, military, and regional agencies

### If necessary, will contact and work with Nuclear Regulatory Commission

### Main responsibilities

#### Assess hazards

#### Keep public informed

#### Recommend emergency action to command team to minimize hazards

## EPA Emergency Response Team (ERT)

### Established under National Contingency Plan

### Objective – To act as or advise On-Scene Coordinator and Regional Response Teams on environmental issues related to

#### Spill containment

#### Cleanup

#### Damage assessment

### Provides expertise in

#### Biology

#### Chemistry

#### Hydrology

#### Engineering of environmental emergencies

### Provides support to full range of emergency response actions

### Can bring in special equipment and experienced responders

### Can provide On-Scene Coordinator or lead responder with advice

## Other Federal Resources

### Department of Homeland Security (DHS)

### Department of Energy (DOE)

### Department of Justice (DOJ)

### Military assets

Learning Activity 6-1: Have students complete Learning Activity 6-1 “Determine characteristics of hazardous materials.”

Review Question: What information that should be provided to an emergency response center?

See p. 249 of the manual for answers.

Section VI:	Learning Activities

# Learning Activities

p. 252	Objective 6 – Determine characteristics of hazardous materials.

## Learning Activity 6-1

### For this activity, students will determine characteristics of hazardous materials

### This activity can be found in Learning Activity 6-1 on the curriculum flash drive

Section VII:	Summary and Review

# SUMMARY AND REVIEW

## Chapter Summary

### Research materials are wide-ranging and can vary from written reference resources to web-based resources.

### The prudent haz mat response organization will research what reference manuals and materials are most beneficial.

### Organizations should train with these resources to ensure that when called upon, the research specialist will be able to produce quality data that will assist the Command team in making a well-informed, risk-based decision.

## Review Questions

### What are the responsibilities of a technical research specialist at a haz mat incident? (227)

### What are the two elements of chemical research? (231)

### What written technical references can be used to gather hazardous materials information? (236-243)

### What chemical databases may be used to gather hazardous materials information? (244-246)

### What information that should be provided to an emergency response center? (249)