Skill Sheet 8-1

Objective 7	Resupply an ARFF vehicle with water. [NFPA® 1003, 4.3.8]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will resupply an ARFF vehicle with water from one or more water sources.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures. Firefighters must properly wear appropriate PPE when performing this skill.

Resources

| - Apparatus (ARFF truck and tender) - Manufacturer’s operators manual - Water source (hydrant and overhead fill location) - Supply hose | - Hose adapters - Hydrant wrench - Proximity Fire Fighting Protective Clothing; Structural Fire Fighting Protective Clothing; or Station Uniform, ear and eye protection, and gloves |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 8-1
Evaluation Checklist

Objective 7	Resupply an ARFF vehicle with water. [NFPA® 1003, 4.3.8]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Water Resupply – Water Tender (Mobile Water Supply) |  |  |
| 1. | Position water tender to resupply ARFF apparatus. |  |  |
| 2. | Connect supply hose from water tender to ARFF apparatus water fill connection. |  |  |
| 3. | Open ARFF apparatus tank pressure relief devices, if necessary. |  |  |
| 4. | Open ARFF apparatus tank fill valve. |  |  |
| 5. | Engage tender water pump. |  |  |
| 6. | Open tender discharge valve. |  |  |
| 7. | Monitor water resupply process. |  |  |
| 8. | When ARFF apparatus water tank is full, shut off tender discharge valve. |  |  |
| 9. | Close ARFF apparatus water tank fill valve. |  |  |
| 10. | Disengage tender water pump. |  |  |
| 11. | Disconnect supply hose from ARFF apparatus. |  |  |
|  | Water Resupply - Overhead Water Fill Point |  |  |
| 1. | Position ARFF apparatus under overhead water fill point for resupply. |  |  |
| 2. | Open ARFF apparatus water tank fill hatch. |  |  |
| 3. | Place overhead water fill hose into ARFF apparatus water tank opening. |  |  |
| 4. | Open overhead water fill point valve. |  |  |
| 5. | Monitor water resupply process. |  |  |
| 6. | When ARFF apparatus water tank is full, shut off overhead fill point valve. |  |  |
| 7. | Close ARFF apparatus water tank fill hatch. |  |  |

|  | Water Resupply - Hydrant |  |  |
| --- | --- | --- | --- |
| 1. | Position ARFF apparatus for resupply. |  |  |
|  | CAUTION: Do not overpressurize tank - see manufacturer’s instructions |  |  |
| 2. | Connect supply hose to ARFF apparatus water fill connection. |  |  |
| 3. | Connect supply hose to hydrant. |  |  |
| 4. | Open ARFF apparatus water tank pressure relief devices, if necessary. |  |  |
| 5. | Open ARFF apparatus water tank fill valve. |  |  |
| 6. | Open hydrant valve. |  |  |
| 7. | Monitor water resupply process. |  |  |
| 8. | When ARFF apparatus water tank is full, shut off hydrant valve. |  |  |
| 9. | Close ARFF apparatus water tank fill valve. |  |  |
| 10. | Close ARFF apparatus water tank pressure relief devices, if necessary. |  |  |
| 11. | Disconnect supply hose from ARFF apparatus and hydrant. |  |  |