Skill Sheet 9-2

Objective 6	Extinguish a fuel spill using an ARFF vehicle turret. [NFPA® 1003, 4.3.2, 4.3.9, 4.3.10]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will extinguish a fuel spill fire using an ARFF vehicle turret.

NOTE: NFPA® 1003 specifies that a fire of at least 2,067 square feet (192 meters squared) should be extinguished in 90 seconds with turret flow of a minimum of 250 gpm (946 L/ min).

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures. Firefighters must properly wear appropriate PPE when performing this skill.

Resources

| - ARFF apparatus equipped with a turret - Fuel spill fire trainer | - Proximity Fire Fighting Protective Clothing or Structural Fire Fighting Protective Clothing |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 9-2 
Evaluation Checklist

Objective 6	Extinguish a fuel spill using an ARFF vehicle turret. [NFPA® 1003, 4.3.2, 4.3.9, 4.3.10]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Deploy the turret as the ARFF apparatus approaches fire. |  |  |
| 2. | Activate the agent pump(s) as required. |  |  |
| 3. | Aim the agent stream above fire. |  |  |
| 4. | Begin discharging agent when the apparatus is within turret range. |  |  |
| 5. | Operate the turret at appropriate pressure and flow using prescribed techniques until fire is extinguished. - Adjust the turret pattern as necessary for distance and fire/fuel conditions. - Interrupt the agent discharge as needed to determine fire/fuel conditions. |  |  |
|  | NOTE: Practice agent conservation. |  |  |
| 6. | Stop discharging when the fire is extinguished. |  |  |