Skill Sheet 9-3

Objective 7	Deploy and operate an ARFF vehicle handline to extinguish a fuel spill fire. [NFPA® 1003, 4.3.1, 4.3.9, 4.3.10]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will deploy and operate an ARFF vehicle handline to extinguish a fuel spill fire.

NOTE: NFPA® 1003 specifies the handline must flow a minimum of 95 gpm (359 L/min) of AFFF extinguishing agent. A 750 square-foot (69.7 meters squared) fire should be extinguished in 90 seconds. Students must open, close, and adjust nozzle flow and patterns.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures. Firefighters must properly wear appropriate PPE when performing this skill.

Resources

| - ARFF apparatus with preconnected hose and attached nozzle - Fuel spill fire trainer | - Proximity Fire Fighting Protective Clothing, including SCBA or Structural Fire Fighting Protective Clothing, including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 9-3 
Evaluation Checklist

Objective 7	Deploy and operate an ARFF vehicle handline to extinguish a fuel spill fire. [NFPA® 1003, 4.3.1, 4.3.9, 4.3.10]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Access the handline. - Unlock the hose reel, if necessary. - Remove security netting/open hoseline compartment. |  |  |
| 2. | Pull the handline. - Fully deploy the handline. - Properly position the handline to avoid kinking or knotting. |  |  |
| 3. | Charge the handline. |  |  |
| 4. | Open the nozzle to bleed air from the handline. |  |  |
|  | CAUTION: Open the nozzle slowly to reduce nozzle reaction. |  |  |
| 5. | Test hose stream pattern. |  |  |
| 6. | Close the nozzle. |  |  |
|  | CAUTION: Close the nozzle slowly to reduce water hammer. |  |  |
| 7. | Advance the handline toward the fire maneuvering around obstacles, as needed. |  |  |
| 8. | Approach the fire. |  |  |
| 9. | Operate the nozzle at appropriate pressure and flow using prescribed techniques until fire is extinguished. |  |  |
|  | NOTE: Practice agent conservation. |  |  |
| 10. | Back away, maintain situational awareness. |  |  |