Skill Sheet 9-5

Objective 9	Attack an interior fire. [NFPA® 1003, 4.3.4, 4.3.7, 4.3.9, 4.3.10]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will attack an interior fire. This skill requires two firefighters, working as a team. Students shall maintain the integrity of the team. Before the skill begins, indicate to students what position to take. Students repeat the skill until each student has served in each position.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures. Firefighters must properly wear appropriate PPE when performing this skill.

NOTE: The method of attack will be dependent on an assessment of the fire conditions.

Resources

| - ARFF apparatus with preconnected handline - Interior live fire trainer | - Means of access (as required) - Proximity Fire Fighting Protective Clothing, including SCBA or Structural Fire Fighting Protective Clothing, including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 9-5 
Evaluation Checklist

Objective 9	Attack an interior fire. [NFPA® 1003, 4.3.4, 4.3.7, 4.3.9, 4.3.10]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Direct Method |  |  |
| 1. | Gain access to aircraft interior. |  |  |
| 2. | Advance the handline toward fire maneuvering around obstacles, as needed. |  |  |
|  | NOTE: Both firefighters should be on the same side of the handline. |  |  |
| 3. | Ensure proper nozzle pattern for fire/fuel conditions prior to opening the nozzle. |  |  |
| 4. | Open the nozzle slowly to avoid nozzle reaction. |  |  |
| 5. | Aim the agent stream at the base of the fire. |  |  |
| 6. | Sweep the agent stream from side-to-side extending beyond the edge of the fire to each side. |  |  |
| 7. | Adjust the nozzle pattern as needed based upon fire/fuel conditions. |  |  |
|  | NOTE: Practice agent conservation. |  |  |
| 8. | Maneuver the handline and nozzle as necessary. |  |  |
| 9. | Stop discharging when the fire is extinguished or the fuel surface has been covered with a foam layer. |  |  |
| 10. | Shut off the nozzle slowly to reduce water hammer. |  |  |
| 11. | Ventilate the fuselage to remove heat and other products of combustion. |  |  |
| 12. | Back away from the fire. Always maintain situational awareness of area for signs of reignition. |  |  |
|  | Indirect Method |  |  |
| 1. | Gain access to aircraft interior. |  |  |
| 2. | Advance the handline toward fire maneuvering around obstacles, as needed. |  |  |
|  | NOTE: Both firefighters should be on the same side of the handline. |  |  |
| 3. | Ensure proper nozzle pattern for fire/fuel conditions prior to opening the nozzle. |  |  |
| 4. | Open the nozzle slowly to avoid water hammer. |  |  |
| 5. | Aim the agent stream above the fire or deflect it off of aircraft surfaces. |  |  |
| 6. | Sweep the agent stream from side-to-side extending beyond the edge of the fire to each side. |  |  |
| 7. | Adjust nozzle pattern as needed based upon fire/fuel conditions. |  |  |
|  | NOTE: Practice agent conservation. |  |  |
| 8. | Maneuver the handline and nozzle as necessary. |  |  |
| 9. | Stop discharging when the fire is extinguished or the fuel surface has been covered with a foam layer. |  |  |
| 10. | Shut off the nozzle slowly to reduce water hammer. |  |  |
|  | CAUTION: Close the nozzle |  |  |
| 11. | Ventilate the fuselage to remove heat and other products of combustion. |  |  |
| 12. | Back away from the fire. Always maintain situational awareness of area for signs of reignition. |  |  |
|  | Combination Method |  |  |
| 1. | Gain access to aircraft interior. |  |  |
| 2. | Advance the handline toward fire maneuvering around obstacles, as needed. |  |  |
|  | NOTE: Both firefighters should be on the same side of the handline. |  |  |
| 3. | Ensure proper nozzle pattern for fire/fuel conditions prior to opening the nozzle. |  |  |
| 4. | Open the nozzle slowly to avoid water hammer. |  |  |
| 5. | Open the dry chemical nozzle, if so equipped. |  |  |
| 6. | Aim the agent stream(s) at the base of the fire. |  |  |
| 7. | Sweep the agent stream(s) across the fuel’s surface to one side. |  |  |
| 8. | Then raise and aim the agent stream(s) above the fire or deflect it off of surfaces while sweeping to the other side of the fire. |  |  |
| 9. | Sweep the agent stream(s) back across the base of the fire then back above. - Repeat as necessary. - Adjust nozzle pattern as needed based upon fire/fuel conditions. |  |  |
|  | NOTE: Practice agent conservation. |  |  |
| 10. | Maneuver handline and nozzle as necessary. |  |  |
| 11. | Stop discharging when the fire is extinguished or the fuel surface has been covered with a foam layer. |  |  |
| 12. | Close the nozzle(s) slowly to reduce water hammer. |  |  |
| 13. | Ventilate the fuselage to remove heat and other products of combustion. |  |  |
| 14. | Back away from the fire. Always maintain situational awareness of area for signs of reignition. |  |  |