Skill Sheet 10-2

Objective 8	Perform an engine compartment inspection and routine preventative maintenance of an ARFF apparatus. [NFPA® 1002, 9.1.1]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will perform engine compartment inspection and routine preventive maintenance. This skill sheet assumes students are performing this inspection and routine maintenance following the in-cab inspection.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures. Firefighters must properly wear appropriate PPE when performing this skill.

Resources

| - Fire service apparatus - Well ventilated inspection area - Wheel chocks - Manufacturer’s operators manual - Basic automotive tool kit - Manufacturer’s approved maintenance items: - Engine crankcase oil - Antifreeze - Power steering fluid - Brake fluid - Windshield washer fluid | - Mixture of baking soda and water - Wire brush - Clear water - Bucket - Clean cloths - Apparatus maintenance logs/forms - Writing instrument - Station wear or coveralls - Gloves - Hearing protection - Eye protection - Helmet |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 10-2 
Evaluation Checklist

Objective 8	Perform an engine compartment inspection and routine preventative maintenance of an ARFF apparatus. [NFPA® 1002, 9.1.1]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | NOTE: Ensure vehicle is properly prepared and all personnel are clear before performing this skill. Remember that inspection and maintenance must be appropriately documented per local SOP/Gs. |  |  |
|  | Prepare for Inspection |  |  |
|  | NOTE: Check the apparatus operator’s manual to determine whether the transmission and power steering fluid levels must be checked while the vehicle is running at normal operating temperature. If this is the case, perform these checks now (directly after the in-cab inspection), before shutting off the apparatus. |  |  |
|  | CAUTION: Diesel exhaust may contain up to 100 harmful chemicals and compounds. Do not run these engines in unvented areas for any period of time. |  |  |
| 1. | Shut down the apparatus. Chock wheels, if necessary. |  |  |
|  | Leaks |  |  |
| 2. | Inspect the condition of all hoses and hydraulic lines for leaking fluids. These include antifreeze, water, windshield wiper fluid, oil, fuel, transmission fluid, hydraulic fluid, power steering fluid, and/or battery fluid. |  |  |
|  | NOTE: Checking for leaks should be done throughout inspection and maintenance. |  |  |
|  | Engine (Crankcase) Oil |  |  |
| 3. | Determine the oil level by using the dipstick. |  |  |
|  | NOTE: This task should be performed when the apparatus is parked on a flat/level surface and oil has had a chance to settle. |  |  |
| 4. | Add oil through the fill port on the engine block, if necessary. Consult the operator’s manual for the proper type of oil and fill parameters. |  |  |
|  | Engine Air Filter |  |  |
| 5. | Inspect the air intake system for signs of damage or dirt buildup. Do not remove filter for inspection. |  |  |
| 6. | Change the air filter if the air filter restriction gauge indicates excessive resistance per the manufacturer’s recommendations. Be sure to follow instructions in the operator’s manual and fire department policy. |  |  |
|  | NOTE: The air filter restriction shall be monitored per NFPA® 1911 (2012) 7.7.5. |  |  |
|  | Exhaust System |  |  |
| 7. | Inspect the exhaust system for damage, recording any damage found on inspection form. |  |  |
| 8. | Test the rain cap on the exhaust system (if so equipped), ensuring that it operates freely. |  |  |
|  | Radiator Coolant |  |  |
|  | WARNING: Check coolant levels when the engine and radiator system are cool. |  |  |
| 9. | Determine whether the antifreeze is at the proper level mark inside the reservoir. - Remove the cap on the antifreeze fill port or look through the sight glass, if one is provided. - Read the coolant level correctly. |  |  |
|  | NOTE: Some vehicles’ coolant reservoirs have two marks: one for when the engine is hot and the other for when it is cool. |  |  |
| 10. | Add coolant, using the type approved by apparatus manufacturer, until amount reaches proper level mark per operator’s manual guidelines. |  |  |
| 11. | Check the radiator hoses, recording any leaks or undue wear in apparatus log or on maintenance form. |  |  |
| 12. | Remove any debris, such as leaves or trash, resting against the radiator or air intake. |  |  |
|  | Cooling Fan |  |  |
|  | WARNING: Some engine cooling fans activate automatically without warning. Use caution when working near the fan. |  |  |
| 13. | Inspect the cooling fan, recording any cracks or missing blades in apparatus log or on maintenance form. |  |  |
|  | Windshield Washer Fluid |  |  |
| 14. | Check the windshield washer fluid level. |  |  |
| 15. | Add windshield washer fluid if tank is less than half full. |  |  |
|  | Battery Condition |  |  |
|  | WARNING: Batteries give off explosive hydrogen gas. Ventilate area when servicing. Wear safety goggles and acid resistant gloves. Exploding batteries can injure or kill. |  |  |
|  | NOTE: Depending on the design of the vehicle, the batteries may be located in the engine compartment or in a separate compartment elsewhere on the vehicle. Most modern vehicle batteries are the sealed type that require no internal inspection by the driver/operator. |  |  |
| 16. | If the apparatus has unsealed batteries, carefully remove the caps and check the electrolyte (water) level. |  |  |
| 17. | Add distilled water, or water recommended by the manufacturer, to cells if the electrolyte level is low. |  |  |
| 18. | Check all battery connections. - Tighten any loose connections. - Clean away any corrosion around terminals with a mixture of baking soda and water poured on the connections, scrubbed with a wire brush, and rinsed with clear water. If batteries are washed, dry batteries to prevent parasitic current. - Clean road debris, dirt, dust, moisture from the top of the batteries to prevent any ‘bleed’ of current from terminal to terminal that can result in electronics issues. |  |  |
| 19. | Check the battery tie-downs, ensuring that the battery is held firmly in place. |  |  |
| 20. | Check the built-in battery charger if the apparatus is so equipped, ensuring proper operation. |  |  |
|  | Automatic Transmission Fluid Level |  |  |
|  | NOTE: Depending on the manufacturer’s recommendations, it may be necessary to check the transmission fluid level after the vehicle has been driven and shifted through all forward gears, and while the vehicle is running. |  |  |
| 21. | Check the automatic transmission fluid level and condition on the dipstick, sight glass, and/or electronic readout according to manufacturer instructions and fire department policy. |  |  |
| 22. | Add fluid to the automatic transmission if the reading on the dipstick or readout indicates that the fluid is low. Be sure to add the proper type and amount per operator’s manual. |  |  |
|  | Power Steering Fluid Level |  |  |
|  | NOTE: Perform the power steering fluid level check now if you did not do it earlier before shutting down the apparatus. |  |  |
| 23. | Check the manufacturer’s indicator marks to determine the power steering fluid level according to manufacturer instructions and fire department policy. Report any drop in levels. |  |  |
| 24. | Add fluid if the reading indicates that the fluid is low. Make sure to add the proper type and amount according to the operator’s manual. |  |  |
|  | CAUTION: Be careful not to overfill the reservoir because damage can occur to the system. |  |  |
|  | Brake Fluid (Hydraulic Brake Systems) |  |  |
| 25. | Check the level of the brake fluid in the master brake cylinder, following the procedure outlined in the operator’s manual. |  |  |
| 26. | Add fluid if the fluid is low. Be sure to add the proper type and amount per operator’s manual. |  |  |
|  | Air System |  |  |
| 27. | Check for leaks in the system. - With the air system at normal operating pressure and the engine shut off, walk around the apparatus and listen for leaks. - On air tanks not equipped with automatic drains, any accumulated condensation should be drained. If the wet tank has excessive amounts of water the air dryer desiccant may need to be replaced and/or the system needs to be serviced by an AHJ approved Emergency Vehicle Technician. |  |  |
|  | Belts |  |  |
|  | WARNING: Never attempt to check the belts while the vehicle’s engine is running. This could result in severe injuries or entrapment in the belt and pulley(s). |  |  |
| 28. | Check all engine compartment belts for tightness and excessive wear. These belts include water pump, air compressor, fan, alternator, etc. |  |  |
|  | NOTE: You should be familiar with the proper feel for tightness of each belt when it is properly adjusted. Most engines have multiple drive pulleys. |  |  |
|  | Electrical Wiring |  |  |
| 29. | Check the electrical wiring in the engine compartment. - Look for frayed, cracked, loose, or otherwise worn wiring. - Record any wiring problems in the apparatus log or on maintenance forms. - Refer any wiring problems to a mechanic for correction. |  |  |
|  | Steering Linkage |  |  |
| 30. | Check the general condition of the steering linkage. - Check for looseness and free play. Report any excessive free play. - Check for missing fasteners. |  |  |
|  | Fuel System |  |  |
| 31. | Store the apparatus with a minimum fuel tank level required by local SOP/Gs. |  |  |
| 32. | Check for any leaks. |  |  |
| 33. | Check the fuel system filters for any indication of contamination, water or debris via the filter sight bowls. |  |  |
| 34. | Inspect fuel tank mounting straps for security and corrosion. |  |  |
|  | Documentation |  |  |
| 35. | Document the inspection and any maintenance actions and report any deficiencies per local policy. |  |  |