Skill Sheet 10-3

Objective 9	Perform an in-cab operational inspection of an ARFF apparatus. [NFPA® 1002, 9.1.1]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will perform an in-cab operational inspection.  Remind students to always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures. Firefighters must properly wear appropriate PPE when performing this skill.

Resources

| - Fire service apparatus - Wheel chocks - Manufacturer’s operators manual - Writing instrument - Apparatus maintenance log/forms | - Station wear or coveralls - Gloves - Hearing protection - Eye protection |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 10-3 
Evaluation Checklist

Objective 9	Perform an in-cab operational inspection of an ARFF apparatus. [NFPA® 1002, 9.1.1]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | NOTE: This skill sheet provides general guidelines performing an in-cab operational inspection. Not all equipment contained on this list will be found on every piece of apparatus. Inspect the items that are on that particular apparatus. Always follow manufacturer’s recommendations and local SOP/Gs. |  |  |
|  | Cab |  |  |
| 1. | Park the apparatus outdoors, if weather permits. |  |  |
| 2. | Set up the inspection area. If indoors, be sure that proper ventilation equipment is in place or doors are open to vent vehicle exhaust. |  |  |
|  | CAUTION: Diesel exhaust may contain up to 100 harmful chemicals and compounds. Do not run these engines in unvented areas for any period of time. |  |  |
| 3. | Chock the vehicle’s wheels. |  |  |
| 4. | Mount the vehicle cab safely. |  |  |
| 5. | Check the seatbelts/restraints. - Be sure that they are securely mounted and operate freely without binding. - Make certain that the webbing is not damaged, cut, or frayed. - Check that the buckles open and close freely. |  |  |
| 6. | Check that the mirrors are not missing or broken. |  |  |
| 7. | Make sure that the tilt/telescopic steering wheel is in a suitable position and is functioning correctly. |  |  |
| 8. | Adjust the seat and mirrors. |  |  |
| 9. | Turn off all accessory electrical switches. |  |  |
| 10. | Start the vehicle. |  |  |
| 11. | Run the engine at low idle per local SOP/Gs until it has warmed to its operating temperature. |  |  |
|  | Dashboard Gauges |  |  |
| 12. | Make sure that all gauges below are functioning in the normal operating range: - Speedometer/odometer - Tachometer - Oil pressure - Fuel gauge - Ammeter and/or voltmeter - Air pressure - Coolant temperature - Vacuum gauge - Hydraulic pressure gauge - Transmission oil temperature gauge - Engine/Pump hour gauge - DEF tank full (2010 and newer SCR equipped diesel engines) - DPF indicator lamp OFF (2007 and newer diesel engines) - Extinguishing agent gauges |  |  |
| 13. | Check that the speedometer is at or very near zero with the apparatus parked. |  |  |
|  | NOTE: If the speedometer shows anything above zero, the gauge is defective or the apparatus is in pump gear. |  |  |
| 14. | Make sure that the fuel gauge reads at least three quarters full or in accordance with departmental SOP/Gs. |  |  |
| 15. | Check that all other gauges register within limits specified in operator’s manual. |  |  |
|  | NOTE: These limits are typically graphically noted directly on the face of the gauge. |  |  |
|  | Control Operations |  |  |
| 16. | Briefly operate all controls in the cab, checking each system below: - Electrical equipment switches - Turn signal switches - High beam headlight switches - Heating and air-conditioning controls - Radio controls - Public address systems (if so equipped) - Audible warning device controls (sirens, auto warning horns, air horns, back-up alarms, seat belt fastening indicators, door open indicators, etc.) CAUTION: Before testing audible warning devices, don appropriate hearing protection. Do not test the operation of audible warning devices indoors or if anyone is standing in front of or near the apparatus. This can cause hearing damage to that person. Test these devices when no one else is in a position to be harmed. - Controls for any computer equipment in the cab (mobile data terminal [MDT], mobile computer terminal [MCT], etc.) - Windshield wiper controls (check fluid reservoir in cab) - Window defroster controls - Load management system - Pump - Turret controls - Turret valves - Drive system interlock control |  |  |
|  | Sterring Wheel |  |  |
| 17. | Check steering wheel free play, noting in apparatus log or on inspection form insufficient or excessive free play. Schedule repair with a certified mechanic if there is inappropriate free play (excess play that does not result in the actual movement of the vehicle’s front tires). |  |  |
|  | NOTE: If the apparatus is equipped with power steering, the engine must be running for the ‘freeplay’ inspection. |  |  |
|  | Air Brake System |  |  |
| 18. | With engine off, wheels chocked, transmission in neutral, full air tanks, and parking brake released, press brake pedal to floor. |  |  |
|  | NOTE: If the apparatus is in service it may need a shoreline in case an alarm comes in during the test. Keep in mind the amount of time for air pressure build-up from above. |  |  |
| 19. | Note any sounds of air leaking, position of needles. After one minute, air pressure should not drop more than 3 psi (20 kPa) for straight-chassis vehicles or 4 psi (30 kPa) for tractor-drawn aerial apparatus. |  |  |
| 20. | With engine off and master switch on, pump brake pedal continually to lower air tank pressure. Warning light and buzzer should activate before 60 psi (420 kPa). |  |  |
| 21. | After Step 20 is complete, with parking brake released, continue to pump brake pedal until air brake control sets automatically by the button popping out. |  |  |
|  | NOTE: This should occur between 20 and 40 psi (140 and 280 kPa). |  |  |
| 22. | Start engine and let air pressure build up to normal. |  |  |
|  | NOTE: This should build to 85-100 psi (595-700 kPa) in 45 sec at idle. |  |  |
| 23. | When the needle stops climbing, the air compressor has cut out. |  |  |
|  | NOTE: This should occur between 120 and 130 psi (840-910 kPa). |  |  |
| 24. | Apply the brake pedal, lowering the pressure in the tanks by 5 to 10 psi (35-70 kPa) increments stopping at 85 psi (595 kPa). Ensure the air gauge needle begins to indicate a pressure increase following the test. |  |  |
|  | Documentation |  |  |
| 25. | Document the inspection and any maintenance actions, and report any deficiencies per local policy. |  |  |