Skill Sheet 10-4

Objective 10	Start, idle, and shut down an ARFF apparatus. [NFPA® 1002, 9.1.2]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will start, idle, and shut down a fire service apparatus. Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all procedures. Firefighters must properly wear appropriate PPE when performing this skill.

Resources

| - Fire service apparatus - Manufacturer’s operator’s manual | - Station wear or coveralls - Hearing protection or Headset |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the candidate has completed the skill sheet, write comments below.

Evaluator/Candidate Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 10-4 
Evaluation Checklist

Objective 10	Start, idle, and shut down an ARFF apparatus. [NFPA® 1002, 9.1.2]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | NOTE: Ensure vehicle is properly prepared and all personnel are clear before performing this skill. |  |  |
| 1. | Disconnect all external electrical cords, air hoses, or exhaust system hoses from the apparatus. |  |  |
| 2. | Mount the apparatus, using appropriate steps and handrails. - Don safety restraints. - Ensure all apparatus occupants are wearing safety restraints and hearing protection. |  |  |
| 3. | Check that the parking brake is set. |  |  |
| 4. | Turn on battery switch. |  |  |
| 5. | Turn on the ignition switch; wait until vehicle computers and systems go through normal start-up. |  |  |
| 6. | Operate the starter control following manufacturer’s recommendations. |  |  |
| 7. | Allow engine to idle and observe the apparatus gauges. - Shut down the engine immediately if oil pressure gauge does not fall within manufacturer’s recommendations within 5 to 10 seconds. - Check ammeter and/or voltmeter to make sure that electrical system is operating/charging properly. - Check air pressure gauge to ensure that adequate pressure is built up to release parking brake and operate service brake. |  |  |
| 8. | Shut off the engine following manufacturer’s procedures. |  |  |
| 9. | Turn the battery switch to the OFF position. |  |  |
| 10. | Reconnect external electrical cords, air hoses, or exhaust system hoses to the apparatus. |  |  |