Chapter 1

Introduction and Review of Awareness, Operations, and Technician-Level Competencies

Lesson Goal

After completing this lesson, the student shall be able to describe the competencies for the Awareness-, Operations-, and Technician-Level responders.

Objectives

Upon successful completion of this lesson, the student shall be able to:

1.	Describe NFPA® 472 competencies for the Awareness-Level responder. [NFPA® 472, 7.2.1.1]

2.	Identify NFPA® 472 competencies for the Operations-Level responder. [NFPA® 472, 7.2.1.1, 7.2.2.1(1-5), 7.2.5.1, 7.2.5.2, 7.2.5.2.2, 7.2.5.3, 7.2.5.4, 7.3.5.5]

3.	Explain NFPA® 472 competencies for the Technician-Level responder.

Instructor Information

This is the lesson covering the hazardous materials competencies for Awareness-, Operations-, and Technician-Level personnel. This lesson also describes the problem-solving process and how it relates to hazardous materials incidents.

Important instructor information is provided in shaded boxes throughout the lesson plan. Carefully review the instructor information before presenting the lesson.

Methodology

This lesson uses lecture and discussion. The level of learning is comprehension.

Outline Contents

| Section | Outline Pages | Text Pages |
| --- | --- | --- |
| Section I: Awareness-Level Competencies | 5 | 10 |
| Section II: Operations-Level Competencies | 33 | 42 |
| Section III: Technician-Level Competencies | 61 | 62 |
| Section IV: Summary and Review | 70 | 70 |

Audiovisuals

- Visuals 1.1 to 1.80 (PowerPoint® Presentation)

- Easel pad and marker

Evaluation

- Chapter 1 Quiz

- Chapter 1 Test

Presentation Tools

Interactive Objects allow instructors to present information a piece at a time by clicking on hot spots in a larger image.

Each object is labeled in the Lesson Outline and indicated in the PowerPoint® presentation by a RED arrow in the top left corner of the image. This lets instructors know to use the mouse to explore all of the information on the slide.

After all of the information is displayed a reset arrow will appear in the bottom right corner of the image. Instructors can use this to remove the text and quiz students on content just covered or simply move on to the next slide.

To move to the slide after an interactive object, select the Click for next slide arrow on the left side of the slide.

Videos are used as discussion starters or to illustrate a concept or process discussed in the chapter.

These are labeled in the Lesson Outline and indicated in the PowerPoint® presentation by the Click image to play arrow on the left side of the slide.

Section I:	Awareness-Level Competencies

# Awareness-level competencies

pp. 10-41	Objective 1 – Describe NFPA® 472 competencies for the Awareness-Level responder.

Review Question: What are the three main Awareness-Level competencies?

See p. 10 of the manual for answers.

## Awareness-Level Competencies

### Analyze the incident to detect the possible presence of a hazardous material

### Survey the incident from a safe location to identify the chemical’s:

#### Name

#### United Nations/North American (UN/NA) number

#### Type of placard

#### Other distinctive markings

### Collect information from the current edition of the Emergency Response Guidebook (ERG)

Instructor Note: Explain to students that there are many response models that can be used to measure benchmarks during a hazardous materials incident. In this manual, the APIE method will be used. APIE stands for:

 Analyze the incident.
 Plan the initial response.
 Implement the response.
 Evaluate the response.

## Analyzing the Incident

### Occupancy types, locations, and preincident surveys

#### May provide first clue that hazardous materials may be involved in incident

#### Items identified during preincident surveys

##### Exposures

##### Types, quantities, and locations of hazardous materials in area

##### Dangers of hazardous materials

##### Building features

##### Site characteristics

##### Possible access/egress difficulties

##### Twenty-four hour telephone numbers of responsible parties and site experts

Review Question: What are a few occupancies that may hold significant quantities of hazardous materials?

See pp. 12-13 of the manual for answers.

#### Occupancies that may hold significant quantities of hazardous materials

##### Fuel storage facilities

##### Gas/service stations and convenience stores

##### Paint supply stores

##### Plant nurseries, garden centers, and agricultural facilities

##### Pest control and lawn care companies

##### Medical facilities

##### Photo processing laboratories

##### Dry cleaners

##### Plastics and high-technology factories

##### Metal plating facilities

##### Mercantile concerns

##### Chemistry laboratories in educational facilities

##### Print shops

##### Industrial and utility plants

##### Port shipping facilities

##### Treatment storage disposal facilities

#### Residential occupancies are not exempt from danger

#### First responders should be familiar with types of hazardous materials that come through jurisdiction

#### Certain occupancies are more likely to be targeted for terrorist attacks

### Container shapes

#### Can alert responders to presence of hazardous materials

#### Can provide useful information about materials inside

### UN Recommendations

#### Adopted by United States, Canada, and Mexico

#### Provides a uniform basis for development of harmonized regulations for all modes of transport

#### Facilitates trade and the safe and efficient transport of hazardous materials

#### Provides:

##### Minimum requirements applicable to transport of hazardous materials by all modes of transport

##### Comprehensive criteria-based classification system for substances that pose significant hazard in transportation

##### Standards for packaging and multimodal tanks used to transport hazardous materials

##### System for communicating hazards of substances in transport through hazard communication requirements

#### Addresses labeling and marking of packages, placarding of tanks and freight units, and documentation and emergency response information required to accompany each shipment

Review Question: What are the nine UN hazard classes?

See pp. 17-18 of the manual for answers.

#### Includes nine hazard classes

##### Class 1 – Explosives

##### Class 2 – Gases

##### Class 3 – Flammable liquids

##### Class 4 – Flammable solids, substances liable to spontaneous combustion, substances that emit flammable or toxic gases on contact with water

##### Class 5 – Oxidizing substances and organic peroxides

##### Class 6 – Toxic and infectious substances

##### Class 7 – Radioactive materials

##### Class 8 – Corrosive substances

##### Class 9 – Miscellaneous dangerous substances and articles

#### Includes a system of four-digit identification numbers

##### Displayed on placards, labels, orange panels, and/or markings in association with materials being transported in cargo tanks, portable tanks, tank cars, or other containers and packages

##### Must be displayed on bulk containers either within placard or stand alone in orange box with black lettering

##### In North America, must be displayed on:

          - Rail tank cars

          - Cargo tank trucks

          - Portable tanks

          - Other bulk packaging

          - Transport vehicles or freight containers containing large quantities of hazardous materials

          - Certain nonbulk packages

#### Forms the basis for U.S. Department of Transportation (DOT) regulations

### DOT placards and labels

#### Placards

##### Diamond-shaped, color-coded signs provided by shippers to identify materials in transportation containers

##### Different for each of nine hazard classes

##### Found on:

          - Bulk packages

          - Rail tank cars

          - Cargo tank vehicles

          - Portable tanks

          - Unit load devices containing hazardous materials over 640 cubic feet (18 m3) in capacity

          - Certain nonbulk containers

##### Required on any container transporting any quantity of the materials listed in Table 1.4 (p. 21 of the manual) or freight containers containing 1,001 pounds (454 kg) aggregate gross weight or more of hazardous materials listed in Table 1.5 (p. 21 of the manual)

##### Not required when gross weight of any of materials listed in Table 1.5 (p. 21 of the manual) is less than 1,001 pounds (454 kg) for any one freight container

Instructor Note: Refer students to Table 1.6 on pp. 22-27 of the manual for an explanation of the DOT hazard classes and subdivisions, the Hazardous Material Regulation (HMR) references for each, and their association placards.

Instructor Note: Discuss with students the “Placard Information” information box on pp. 19-20 of the manual.

#### Labels

##### Provide same information as vehicle placards, but are placed on individual packages

##### 3.9-inch (100 mm), square-on-point diamonds

##### May or may not have written text that identifies hazardous material within packaging

Instructor Note: Refer students to Table 1.7 on pp. 28-29 of the manual for examples of unique DOT labels.

##### Packages may include primary and secondary label

### Transport Canada and the Dangerous Goods Act

#### Govern transportation placards, labels, and markings in Canada

#### Based on UN Recommendations and have same nine hazard classes

#### Differences from U.S. placards, labels, and markings

##### Most Canadian transport placards do not have any signal words on them

##### Labels and markings may be in both English and French

##### Canada requires a unique placard for anhydrous ammonia

### Mexican placards, labels, and markings

#### Based on UN Recommendations and have same hazard classes and subdivisions

#### May have text in Spanish

#### Differences from U.S. Hazardous Material Regulation

##### Official Mexican standards do not authorize the use of dangerous placard since NOM-004 does not include provisions for its use; however, peligroso (dangerous) placards may still be seen in Mexico

##### Package markings are consistent except that proper shipping name is provided in Spanish in addition to English

##### Hot mark used for elevated temperature materials in U.S. is not authorized in Mexico

##### Mexican regulations do not require marine pollutants mark for surface transportation

##### Mexican standards incorporate provisions for consumer commodities but do not authorize use of ORM-D description as a package marking

##### The Mexican standard regarding classification of flammable liquids does not incorporate provisions for combustible liquids

### Other transportation vehicle markings

#### Company names

#### Logos

#### Specific tank colors

#### Stenciled commodity names

#### Manufacturers’ specification plates

### NFPA® 704 system

#### Provides widely recognized method for indicating presence of hazardous materials at commercial, manufacturing, institutional, and other fixed-storage facilities

#### Commonly required by local ordinances for all occupancies that contain hazardous materials

#### Designed to alert emergency responders to health, flammability, instability, and related hazards

#### NOT designed for certain situations

##### Transportation

##### General public use

##### Nonemergency occupational exposures

##### Explosives and blasting agents, including commercial explosive materials

##### Chronic health hazards

##### Biological agents and other similar hazards

#### Uses a rating system of numbers from 0 to 4

##### 0 indicates minimal hazard; 4 indicates a severe hazard

##### Assigned to three categories: health, flammability, and stability

##### Arranged on a diamond-shaped marker or sign

##### Health – Blue background

##### Flammability – Red background

##### Instability – Yellow background

##### Special hazards – White background in six o’clock position

#### Provides very useful information, but has limitations

##### No specific quantities

##### No specified location on buildings

### Other markings and colors

#### May indicate presence of hazardous materials at fixed facilities, on pipelines, on piping systems, and on other containers

#### May be simple or complicated

#### May include identification numbers that correspond to site or emergency plans

#### Common specialized systems used in North America

##### Common hazardous communication labels

##### International Organization for Standardization (ISO) safety symbols

##### Globally Harmonized System symbols

##### Military markings

##### Pipeline identifications

##### Signal words

##### Chemical Abstracts Service (CAS) numbers

##### American Petroleum Institute (API) markings

##### Pesticide labels

##### Color codes

### Written resources

#### Shipping papers

##### Included with shipments of hazardous materials

##### Provided through bill of lading, waybill, etc.

##### Location and type of paperwork change according to mode of transport

Review Question: What information should be included on shipping papers?

See p. 34 of the manual for answers.

##### Should include:

          - Identification number (UN or NA)

          - Proper shipping name of material

          - Hazard class represented by material

          - Packing group assigned to material

          - Quantity of material

#### May be located and examined by responders trained to Operations-Level or higher if deemed safe to do so

#### Train consists

##### Should be located with train crew

##### If crew cannot be located, contact railroad company or engine for train list

##### Will include count and list of train cars from front to back

#### Bills of lading

##### Shipping papers used by trucking industry

##### Indicate origin, destination, route, and product being carried

##### Must be placed in cab of every truck tractor

##### Must be within arm’s reach of driver

##### Establishes terms of contract between shippers and transportation companies

##### Serves as document of title, contract of carriage, and receipt for goods

#### Safety data sheets (SDSs)

##### Detailed information bulletins prepared by manufacturer or importer of a chemical

##### Provides specific information about product

##### Often best sources of detailed information about a particular material

##### Can be acquired from manufacturer of material, supplier, shipper, emergency response center, or facility hazard communication plan

#### Globally Harmonized System (GHS) format

##### Specifies minimum information to be provided on SDSs

##### Used worldwide

##### Sections

##### 1: Identification

##### 2: Hazard(s) identification

##### 3: Composition/information on ingredients

##### 4: First aid measures

##### 5: Fire fighting measures

##### 6: Accidental release measures

##### 7: Handling and storage

##### 8: Exposure controls/personal protection

##### 9: Physical and chemical properties

##### 10: Stability and reactivity

##### 11: Toxicological information

##### 12: Ecological information

##### 13: Disposal considerations

##### 14: Transport information

##### 15: Regulatory information

##### 16: Other information

#### Emergency Response Guidebook (ERG)

##### Used by firefighters, law enforcement, and other emergency services personnel who may be first to arrive at scene involving dangerous goods/hazardous materials

##### Aids emergency responders in quickly identifying general or specific hazards of materials involved in emergency incident and protecting themselves and the general public during the initial response phase of the incident

##### Does not address all possible circumstances that may be associated with a dangerous goods/hazardous materials incident

##### Primarily designed for use at an incident occurring on highway or railroad

#### Chemical Inventory Lists (CILs)

##### Required of all U.S. employers for all hazardous substances

##### Contain information about locations of materials within a facility

##### Other documents that may provide information about hazardous materials at facility

###### Shipping and receiving documents

###### Inventory records

###### Risk management and hazardous communication plans

###### Chemical inventory reports

#### Local Emergency Planning Committees (LEPCs)

##### Good source of information

##### Designed to provide forum for emergency management agencies, responders, industry, and public to work together to evaluate, understand, and communicate chemical hazards in community

##### Develop appropriate emergency plans in case of accidental release of these chemicals

### Senses

#### May be used with caution in detection of a hazardous material

#### Vision – Safest to use

#### Other senses usually require contact too close to be safe

WARNING! Deliberately using the human senses to detect the presence of hazardous materials is both unreliable and dangerous.

Interactive slide; use mouse to show all information

Direct visible evidence that physical and/or chemical actions and reactions are taking place

##### Spreading vapor cloud or smoke

##### Unusual colored smoke

##### Flames

##### Changes in vegetation

##### Container deterioration

##### Containers bulging

##### Sick humans

##### Dead or dying birds, animals, insects, or fish

##### Discoloration of valves or piping

#### Physical changes

##### Processes that do not change elemental composition of materials involved

##### Indications

###### Rainbow sheen on water surfaces

###### Wavy vapors over volatile liquid

###### Frost or ice buildup near a leak

###### Containers deformed by force of accident

###### Activated pressure-relief devices

###### Pinging or popping of heat-exposed vessels

#### Chemical reactions – Evidence:

##### Exothermic heat

##### Unusual or unexpected temperature drop

##### Extraordinary fire conditions

##### Peeling or discoloration of container’s finish

##### Spattering or boiling on unheated materials

##### Distinctively colored vapor clouds

##### Smoking or self-igniting materials

##### Unexpected deterioration of equipment

##### Peculiar smells

##### Unexplained changes in ordinary materials

##### Symptoms of chemical exposure

#### Signs and symptoms of exposure

##### Changes in respiration

##### Changes in level of consciousness

##### Abdominal distress

##### Change in activity level

##### Visual disturbances

##### Skin changes

##### Changes in excretion or thirst

##### Pain

### Monitoring and detection

#### Can be useful in determining presence of hazardous materials as well as concentration(s) present

#### Can be used to determine scope of incident

#### Requires actual contact with hazardous material

#### Outside scope of action for Awareness-Level personnel

## Implementing the Planned Response

Review Question: What are the Awareness-Level responsibilities in implementing the planned response?

See pp. 38-39 of the manual for answers.

### Awareness-Level responsibilities

#### Recognize presence or potential presence of a hazardous material

#### Recognize type of container at site and identify material in container if possible

#### Transmit information to appropriate authority and call for appropriate assistance

#### Identify actions to protect themselves and others from hazards

#### Establish scene control by isolating hazardous area, denying entry, and initiating Incident Command System (ICS)

### Predetermined procedures and emergency response plans

#### Also known as standard operating procedures (SOPs), standard operating guidelines (SOGs), or operating instructions (OIs)

#### Spell out role of Awareness-Level personnel at emergency incidents

#### Emergency response plans – Required by OSHA 29 CFR 1910.120(q)(2) to cover

##### Preincident planning and coordination with outside parties

##### Personnel roles, lines of authority, training, and communication

##### Emergency recognition and prevention

##### Safe distances and places of refuge

##### Site security and control

##### Evacuation routes and procedures

##### Decontamination

##### Emergency medical treatment and first aid

##### Emergency alerting and response procedures

##### Critiques of response and follow-up

##### PPE and emergency response equipment

#### Provide standard set of actions that are core of every hazardous materials incident plan

#### Should have built-in flexibility that allows adjustments

#### Usually initiated by first units on-scene

#### Do not replace size-up decisions based on judgment, evaluation, or command

#### Reduce on-scene confusion

#### Should be standardized, clearly written, and mandated to each member of the organization

#### Establish accountability and increase command and control effectiveness

#### Prevent duplication of effort and uncoordinated operations

### Notification requirements

#### Should be defined in SOPs

#### May be as simple as dialing 
9-1-1

#### May be specific to facility

#### Should include law enforcement if criminal or terrorist activity suspected

### Isolation and denial of entry

#### Two of most important means by which responders can ensure their own safety and safety of others

#### Necessary to protect life safety of everyone involved

#### Prevents spread of hazardous materials through cross contamination

#### Involves physically securing and maintaining scene by establishing isolation perimeters and denying entry to unauthorized persons

#### Includes preventing contaminated or potentially contaminated individuals from leaving scene in order to stop spread of hazardous materials

#### May continue with evacuation, defending in place, or sheltering in place

#### Isolation perimeter

##### Boundary established to prevent access by public or unauthorized persons

##### Can be expanded or reduced as needed

##### Used to control both access and egress from scene

##### If inside – Denying entry and exit from building

##### If outside – Set at surrounding intersections by law enforcement, ropes, cones, or barrier tape

Section II:	Operations-Level Competencies

# operations-level competencies

pp. 42-62	Objective 2 – Identify NFPA® 472 competencies for the Operations-Level responder.

## Analyzing the Incident

### First step in mitigating any emergency incident

### Includes incident priorities, Incident Management System (IMS), and predetermined procedures

### Enables first responders to form overall plan of action

### Should include identification of hazardous material or hazard class, as well as other factors that have effect on situation

Review Question: What are a few questions that should be answered during the initial survey?

See pp. 42-43 of the manual for answers.

### Initial survey should include answers to certain questions

#### Where is incident scene in relation to population and environmental and property exposures?

#### Is incident scene inside a building or outside?

#### What are the hazardous materials?

#### What hazard classes are involved?

#### What quantities are involved?

#### What concentrations are involved?

#### How could material react?

#### How is material likely to behave?

#### Is it a liquid or solid spill or a gas release?

#### Is something burning?

#### What kind of container holds the material?

#### What is the condition of container?

#### How much time has elapsed since incident began?

#### What personnel, equipment, and extinguishing agents are available?

#### Is private fire protection or other help available?

#### What effect can weather have?

#### Are there nearby lakes, ponds, streams, or other bodies of water?

#### Are there overhead wires, underground pipelines, or other utilities?

#### Where are nearest storm and sewer drains?

#### What has already been done?

### Information should be gathered and analyzed through a hazard and risk assessment model such as General Emergency Behavior Model (GEBMO)

### Size-up: Surveying materials and containers involved

#### Assessment of incident conditions and recognition of clues indicating problems and potential problems presented by an incident

#### Mental process of considering all available factors that will affect incident during the course of the operation

#### Used to determine strategies and tactics that are applied to incident during planning and implementation stages

#### Includes hazard and risk assessment

#### Must consider all six sides of the incident – Alpha, Bravo, Charlie, Delta, and top and bottom

#### Frequently complicated by limited information

#### Not just responsibility of Incident Commander (IC)

#### Continuing functions that must be performed constantly by all personnel on-scene

#### May include:

##### Number and types of injuries

##### Occupancy type

##### Type of incident

##### Product and container information if available

##### Location of the incident

##### Equipment and resources responding

##### Time of day

##### Weather

#### Should include evaluation of certain conditions on scene

##### Unusual signs

##### Life hazards

##### Product(s) involved

##### Container types

##### Amount of product involved

##### Product travel or path of fire

##### Actions already taken by people on-scene

### Collecting hazard response information

#### Assists in determining level of risk presented by material itself

#### May involve use of ERG, GEBMO, or plume-modeling software

#### Should help to predict where the hazardous material may be going, given its physical state of matter and environmental conditions present

#### May include use of monitoring and detection devices

### Predicting likely behavior

#### Hazardous materials incidents have common elements

##### Material or materials presenting hazards to people, environment, or property

##### Container or containers that have failed or have potential to fail

##### Exposure or potential exposure to people, environment, and/or property

#### GEBMO

##### Helps first responders predict course of incident and establish guidelines for defensive actions

##### Enables responders to limit effects of a hazardous material

#### Should include answering certain questions

##### How long will harmful exposure exist?

##### What has stressed or is stressing the container?

##### How will stressed container and its material behave?

##### What are harmful effects of contained materials?

Review Question: What pattern do hazardous materials events generally follow?

See p. 46 of the manual for answers.

#### Events follow general pattern or model

##### Stress

###### May be thermal, mechanical, or chemical

###### Type will influence how container will breach or release

##### Breach

###### May be partial or total

###### Based on construction material, type of stress that it is exposed to, and pressure inside container at time that it fails

##### Release – Always involves product and may involve release of energy and container parts

##### Dispersion/Engulf – Patterns are based on laws of chemistry and physics and characteristics of product

##### Exposure/Contact – Can be anything in area

##### Harm – Depends on container, product, energy involved

### Estimating potential harm

#### Should include references such as SDSs and ERG to determine health and physical hazards presented by material

#### Should also include prediction where the hazardous material may be going

#### May include monitoring and detection devices

#### Should include definition of level of response, which can identify level of involvement and necessary resources

#### Level I

##### Within capabilities of fire or emergency services organization

##### Least serious and easiest to handle

##### May pose serious threat to life or property, but not usually

##### Evacuation is limited to immediate area of incident

##### Property owner is typically responsible for cleanup and disposal

#### Level II

##### Beyond capabilities of first responders on scene

##### May be beyond capabilities of first response agency/organization having jurisdiction

##### May require services of formal haz mat response team

#### Level III

##### Typically requires resources from state/provincial agencies, federal agencies, and/or private industry

##### Requires Unified Command

##### Most serious of all hazardous materials incidents

##### May require large-scale evacuation

##### Will most likely not be concluded without assistance of outside agencies

##### Often requires use of specialists from industry and government, sophisticated sampling and monitoring equipment, specialized leak and spill control techniques, and large-scale decontamination

## Planning the Initial Response

### Includes establishment of strategic goals and tactical objectives; selected based on certain criteria

#### Ability to be achieved

#### Ability to prevent further injuries and/or deaths

#### Ability to minimize environmental and property damage within constraints of safety, time, equipment, and personnel

### Risk-based response

#### Strategies are based upon hazards present at the incident

#### Principles

##### Activities that present significant risk to safety of members shall be limited to situations where there is potential to save endangered lives

##### Activities that are routinely employed to protect property shall be recognized as inherent risks to safety of members, and actions shall be taken to reduce or avoid these risks

##### No risk to safety of members shall be acceptable when there is no possibility to save lives or property

### Strategic goals and response objectives

#### Strategic goals – Broad statements of what must be done to resolve an incident

#### Tactical objectives – Specific operations done to accomplish strategic goals

#### Standard strategic goals

##### Isolation

##### Notification

##### Identification

##### Protection

##### Rescue

##### Spill/leak control and confinement

##### Crime scene and evidence preservation

##### Fire control

##### Recovery/termination

#### Prioritized depending on available resources and particulars of incident

### Response options

#### Selection is based on risk to responders, their level of training, and balance between resources required and those available

#### May change during the course of an incident

#### Elements affecting selection

##### Value – Related directly to incident priorities of life safety, incident stabilization, and property and environmental conservation

##### Time – Limited window of opportunity exists to intervene before an incident escalates dramatically

##### Size – Most frequently driven by need to conduct protective action concurrently with incident control operations

Review Question: What are the differences between offensive, defensive, and nonintervention operations?

See pp. 51-52 of the manual for answers.

#### Offensive operations

##### Those where responders take direct action on material, container, or process equipment involved

##### May result in contact with material and therefore require responders to wear appropriate chemical-protective clothing and respiratory protection

##### Some are beyond scope of responsibilities for first responders

#### Defensive operations

##### Those in which responders seek to confine the emergency to a given area without directly contacting hazardous materials involved

##### Selected when one of the following circumstances exists:

###### If facility or Local Emergency Response Plan (LERP) calls for it based on a preincident evaluation of hazards present at site

###### If responders have training and equipment necessary to confine incident to area of origin

#### Nonintervention operations

##### Those in which responders do not take direct actions on actual problem

##### Selected when one or more circumstances exist:

###### Facility or LERP calls for it based on preincident evaluation of hazards present at site

###### Situation is clearly beyond capabilities of responders

###### Explosions are imminent

###### Serious container damage threatens a massive release

##### Actions to be taken

###### Initiate IMS

###### Withdraw to safe distance

###### Report scene conditions to telecommunications center

###### Call for additional resources as needed

###### Isolate hazard area and deny entry

###### Commence evacuation where needed

### PPE requirements

#### May include anything from standard fire fighting protective clothing to chemical-protective clothing or body armor

#### Protects skin, eyes, face, hands, feet, body, head, hearing and respiratory system against a variety of hazardous materials

### Emergency decontamination procedures

#### Process of removing hazardous materials to prevent spread of contaminants beyond a specific area and reduce contamination to levels that are no longer harmful

#### Essential activity that must be considered at any hazardous materials or terrorism incident to ensure safety of emergency responders and public

#### Minimize harmful exposures and reduce or eliminate spread of contaminants

#### Performed at haz mat/WMD incidents to remove hazardous materials from responders, victims, PPE, tools, equipment, and anything else that has been contaminated

#### Required of anything that passes through hot zone

#### Provides victims with psychological reassurance

#### Emergency decontamination

##### Purpose – Remove threatening contaminant from victim as quickly as possible

##### No regard for environment or property protection

##### May be necessary for both victims and rescuers

##### Instances

###### Failure of protective clothing

###### Accidental contamination of emergency responders

###### Heat illness or other injury suffered by emergency workers in the hot zone

###### Immediate medical attention is required

##### Has limitations

#### Technical decontamination

##### Uses chemical or physical methods to thoroughly remove or neutralize contaminants from responders’ PPE and equipment

##### Used on incident victims in situations that are not life-threatening

##### Can be performed by Operations-Level responders under guidance of haz mat technician, SOPs, or allied professional

### Incident action plans

#### Critical to rapid, effective control of emergency operations

#### Well thought out, organized course of events developed to address all phases of incident control within specified time frame that allows least negative action to continue

#### Not necessary for short-term, routine operations

#### Start by identifying strategy that will achieve a solution to problems faced; tactics are then selected to achieve strategy

#### Provide for necessary support resources such as water supply, utility control, or SCBA cylinder filling

#### Essentially tie entire problem-solving process together by stating what analysis has found, what plan is, and how it shall be implemented

Review Question: What are the elements of an incident action plan?

See pp. 55-56 of the manual for answers.

#### Elements

##### Strategies/incident objectives

##### Current situation summary

##### Resource assignment and needs

##### Accomplishments

##### Hazard statement

##### Risk assessment

##### Safety plan and message

##### Protective measures

##### Current and projected weather conditions

##### Status of injuries

##### Communications plan

##### Medical plan

### Site safety plans

#### Required by 29 CFR 1910.120(b)(4)(ii)

#### Must encompass all aspects of response

#### Should include, at a minimum

##### Hazard analysis for each site task and operation found in work plan

##### Employee training assignments to ensure compliance with 29 CFR 1910.120 regulation

##### Personal protective equipment requirement for each specific task

##### Medical surveillance requirements

##### Frequency and types of air monitoring, personnel monitoring and environmental sampling techniques and instruments to be used

##### Site control measures

##### An emergency response plan

##### Confined space entry procedures

##### A spill containment program

## Implementing the Planned Response

### Takes place after incident has been surveyed and haz mat response personnel have analyzed data and different response options in planning phase

### Establishing and enforcing control procedures

#### Isolation perimeter

##### May be composed of inner and outer perimeter

##### May be expanded or reduced in size as needed

##### Determined by outcome of on-site risk assessment or size-up done by IC

##### Used to control access and egress from the incident

#### Hazard control zones

##### Hot zone

###### Area surrounding incident that is potentially very dangerous

###### May be contaminated by chemical warfare agents or may have potential to become contaminated by a released hazardous material

###### Work performed is limited to highly trained and equipped personnel

###### Has established access and egress points to ensure accountability and designated PPE

###### Extends far enough to prevent those outside zone from suffering ill effects from released material, explosion, or other threat

##### Warm zone

###### Area adjoining hot zone and extending to cold zone

###### A buffer between hot and cold zones

###### Where decontamination occurs

###### May be considered to be part of the crime scene

###### Requires PPE, perhaps at a reduced level from hot zone

##### Cold zone

###### Surrounds warm zone

###### Used to carry out all logistical support functions of incident

###### Does not require personal protective clothing

###### Includes command post, donning/doffing area, backup teams, research teams, logistical support, criminal investigation teams, triage/treatment/
rehab, transportation areas, and staging area

#### Establishing control zones

##### May be different for each response agency

##### May change as evidence is processed

### Initiating the Incident Management System

#### Forms framework for control and coordination structure that enables emergency personnel to turn chaos into order and effectively manage any emergency incident

#### Any response organizations in the U.S. that receives any type of federal funding are required to use National Incident Management System – Incident Command System (NIMS-ICS)

#### Command

##### Established by first person on scene or ranking individual of first company on scene

##### Maintained until higher ranking or more extensively trained responder arrives on the scene

##### Can be assumed only by someone having IMS training and trained to at least the Operations-Level

#### Transferring command

##### Person accepting must be capable of assuming command and be willing to accept responsibility

##### Can be accomplished over the radio

##### Can only be transferred to someone who is on scene

##### May take place numerous times as a large incident comes under control

##### Must include as clear a picture of the incident as possible; should be acknowledged by repeating information

##### Must be as smooth and efficient as possible

##### Must be announced to avoid confusion

Instructor Note: Reiterate to students that there is only one IC.

#### IMS positions

##### Hazardous Materials Officer (Branch Director/Group Supervisor)

##### Entry Team Leader

##### Decontamination Team Leader

##### Site Access Control Leader

##### Hazardous Materials Safety Officer

##### Technical Specialist (Hazardous Materials Reference/Science Technical Specialist)

##### Safe Refuge Area Manager

### Implementing the Incident Action Plan

#### Includes achievement of strategic goals and tactics

#### Tactics

##### Two categories – Confinement or containment

##### Establishing scene control zones

##### Calling for additional resources

##### Wearing appropriate PPE

##### Performing decontamination

##### Performing fire extinguishment

#### Must include contact between IC and officers and crew

#### If cannot be achieved, may need to be reassessed

### Preserving evidence

#### Must include establishment of security perimeter with access control point

#### Must include control, identification, and removal of persons

#### To aid in securing and protecting the scene, responders should:

##### Prevent individuals from altering or destroying physical evidence by restricting movement, location, and activity while ensuring and maintaining safety at the scene

##### Identify all individuals at the scene who may be suspects or witnesses and secure and separate them

##### Determine if bystanders are witnesses

##### Exclude unauthorized and nonessential personnel from scene

##### Take note of any type of evidence; taking special care to document and protect transient and trace evidence

##### Consult with law enforcement and/or evidence technicians before disturbing transient or trace evidence

##### Document any items within perimeter that have been disturbed or moved by emergency responders

## Evaluating Progress

### Final aspect of problem-solving process

### If IAP is effective, IC should receive favorable progress reports and incident should stabilize

### If mitigation efforts are failing, plan must be reevaluated and revised

Section III:	Technician-Level Competencies

# technician-level competencies

pp. 62-70	Objective 3 – Explain NFPA® 472 competencies for the Technician-Level responder.

Review Question: What are the NFPA® 472 competencies for the Technician-Level responder?

See pp. 62-69 of the manual for answers.

## Training

### Must come in phases

### Must be gained in Awareness and Operations Levels first

### OSHA standards

#### Occupational Safety and Health Administration (OSHA)

##### Outlines training requirements for employees who work with chemicals

##### Establishes various standards

##### Legally enforces standards

#### Hazardous Waste Operations and Emergency Response (HAZWOPER)

##### Broken down into 17 paragraphs

##### Addresses subjects such as medical monitoring and health and safety plans

##### Paragraph Q refers to emergency response

###### Development of Emergency Response Plan

###### Procedures for handling emergency response

###### Skilled support personnel

###### Specialist employees

###### Training

###### Trainers

###### Refresher training

###### Medical surveillance

###### Chemical protective clothing

###### Postemergency response operations

### NFPA® standards

#### NFPA® 472 outlines competencies relating to haz mat incidents

#### Technician-Level competencies included in NFPA® 472

##### Technician-Level Responders

##### Incident Commanders

##### Specialist Employees

##### Hazardous Materials Officers

##### Hazardous Materials Safety Officers

##### Hazardous Materials Technicians with a Tank Car Specialty

##### Hazardous Materials Technicians with a Cargo Tank Specialty

##### Hazardous Materials Technicians with an Intermodal Tank Specialty

##### Hazardous Materials Technicians with a Marine Tank and Non-Tank Vessel Specialty

##### Hazardous Materials Technicians with a Flammable Liquids Bulk Storage Specialty

##### Hazardous Materials Technicians with a Flammable Gases Bulk Storage Specialty

##### Hazardous Materials Technicians with a Radioactive Material Specialty

### Mission-specific competencies

#### Outlined in NFPA® 472

#### Optional proficiencies that allow community and AHJ to match a task with requirements to perform those tasks

#### Over and above core competencies of each response level

### Training sources

#### Federal agencies

##### Centers for Disease Control and Prevention (CDC)

##### Department of Homeland Security (DHS)

##### Federal Emergency Management Agency (FEMA)

#### State and local agencies

##### State and local health departments

##### State and local emergency management departments

## Analyzing the Incident

### Estimating container damage

#### Requires a knowledge of containers, their potential contents, and associated hazards

#### Technicians should know:

##### Container identification

##### Container construction features

##### Container’s typical contents

##### Capacity estimation

##### Common leak points

### Predicting behavior

#### Requires understanding of chemical and physical properties of substances

#### Requires general knowledge of chemistry and the associated properties of substances

### Resources – Assist responders with product identification

### Estimating scope

#### Requires understanding of chemical and physical properties of a product

#### Determined through use of chemical reference manuals and databases

## Planning the Initial Response

### Requires answers to certain questions

#### What response option will be used?

#### Will mitigation be done offensively or defensively?

#### What equipment and PPE will be needed to support chosen option?

#### What decontamination process will be used?

### Using risk-based response

#### Should be taken with regards to considering protective clothing options, control measures, and decontamination options

#### Variety of models to choose from

### Selecting PPE – Critical at Technician Level because of offensive work

### Implementing technical decontamination procedures

#### Required when leaving hazard area

#### Different levels and methods

### Developing Incident Action Plans

#### Required before implementing a response plan

#### May be in writing

#### Must be developed with incident and hazards in mind

## Response

### Duties of hazardous materials technician

#### May be diverse, based on chosen response model and area of responsibility the technician is assigned

#### Primary duty – To work within structure of Incident Command System

#### Proficiency in use of respiratory protection and protective clothing

#### Knowledge of certain safety procedures

##### Actions necessary when there is a loss of air supply

##### Actions necessary when there is a loss of suit integrity

##### Actions necessary when there may be a responder down in the hot zone

#### Lead or work within a decontamination operation

### Product control

#### Primary objective of hazardous materials technician

#### Includes selection and utilization of proper tools, equipment, and materials

## Evaluating Progress

### Last phase of process

### Ensures that progress is being made and that desired outcome will be achieved

### If determines that expected outcome may not be realized, IC must refer back to IAP and begin problem-solving process again

### Includes effectiveness of decontamination process

## Terminating the Incident

### Written documentation

### Debriefing

### Hazard communication briefing

#### Provides information to personnel concerning signs and symptoms of overexposure to hazardous material(s) involved in incident

#### Must be thoroughly documented

#### Includes

##### Identity of material involved

##### Potential adverse effects of exposure to the material

##### Actions to be taken for further decontamination

##### Signs and symptoms of an exposure

##### Mechanism by which a responder can obtain medical evaluation and treatment

##### Exposure documentation procedures

### Postincident critique

#### Required by OSHA Title 29 CFR 1910.120

#### Identifies operational deficiencies and learning from mistakes

#### Needs to occur as soon as possible after incident and involve all responders

#### Documented to identify those in attendance as well as any operational deficiencies that were identified

Section IV:	Summary and Review

# summary and review

## Chapter Summary

### The Awareness- and Operations-Levels reflect a predominately defensive response posture and are intended to keep the emergency responder away from the product while maintaining a high level of safety.

### The haz mat technician is expected to take an offensive posture and must have a solid background in many areas of this response mode.

## Review Questions

### What are the three main Awareness-Level competencies? (10)

### What are a few occupancies that may hold significant quantities of hazardous materials? (12-13)

### What are the nine UN hazard classes? (17-18)

### What information should be included on shipping papers? (34)

### What are the Awareness-Level responsibilities in implementing the planned response? (38-39)

### What are a few questions that should be answered during the initial survey? (42-43)

### What pattern do hazardous materials events generally follow? (46)

### What are the differences between offensive, defensive, and nonintervention operations? (51-52)

### What are the elements of an incident action plan? (55-56)

### What are the NFPA® 472 competencies for the Technician-Level responder? (62-69)