Skill Sheet 5-5

Objective 14	Inspect SCBA. [NFPA 4.5.1]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will inspect SCBA. Inform students of any time requirements for this skill.

Always follow manufacturer’s recommendations and local standard operating procedures or guidelines (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - SCBA including PASS device | - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 5-5 
Assessment

Objective 14	Inspect SCBA. [NFPA 4.5.1]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Ensure all components of SCBA are present: harness assembly, cylinder, facepiece, hoses, end of service indicators, regulators, and accessories (e.g., PASS device). |  |  |
| 2. | Inspect all components of SCBA for cleanliness and damage. |  |  |
| 3. | Immediately clean dirty components. If damage is found, remove component from service and report to company officer. |  |  |
| 4. | Check that cylinder is full (at least 90% capacity). |  |  |
| 5. | Open the cylinder valve slowly; verify operation of the low-pressure alarm and the absence of audible air leaks. NOTE: On some SCBA, the audible alarm does not sound when the cylinder valve is opened. |  |  |
| 6. | If air leaks are detected, determine if connections need to be tightened or if valves, donning switch, etc. need to be adjusted. Otherwise, equipment with audible leaks due to malfunctions shall be removed from service, tagged, and reported to the company officer. |  |  |
| 7. | Check that gauges and/or indicators (e.g., heads-up display) are providing similar pressure readings. Manufacturer’s guidelines determine the acceptable range. |  |  |
| 8. | Check the function of all modes of the PASS device. |  |  |
| 9. | Don facepiece over the head and securely tighten the straps. |  |  |
| 10. | Test the facepiece for a proper seal and proper operation of the exhalation valve. NOTE: Not all facepieces are designed for a seal check without the regulator being attached and activated. |  |  |
| 11. | Don the regulator and check its function by taking several normal breaths. |  |  |
| 12. | Test bypass and/or purge valve (if applicable). |  |  |
| 13. | Close cylinder. |  |  |
| 14. | Bleed off air line and test low pressure alarm. |  |  |
| 15. | Remove facepiece and prepare all components for immediate reuse. |  |  |
| 16. | Document inspections per local SOPs. |  |  |