Skill Sheet 5-8

Objective 17	Replace an SCBA cylinder. [NFPA 4.3.1]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will replace an SCBA cylinder. Remind students to always follow manufacturer’s recommendations for the specific cylinders their department uses. Students must know the operation of the particular unit they are using.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

CAUTION: Clearing debris and quickly opening the valve can cause damage or failure of the O-ring. Eye protection should be worn.

Resources

| - Replacement cylinder - SCBA including PASS device | - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 5-8
Assessment

Objective 17	Replace an SCBA cylinder. [NFPA 4.3.1]

NOTE: Whether one or two firefighters perform this skill, the firefighter who has easiest access to that part of the assembly should perform the step.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Position the cylinder for easy access either on the ground (one firefighter) or by kneeling down or bending over (two firefighters). |  |  |
| 2. | Fully close the cylinder valve. |  |  |
| 3. | Release the air pressure from high- and low-pressure hoses. |  |  |
| 4. | Disconnect the high-pressure coupling from the cylinder. |  |  |
| 5. | Remove the empty cylinder from harness assembly. |  |  |
| 6. | Inspect the empty cylinder’s valve opening, high pressure hose fitting, and the O-ring for debris. |  |  |
| 7. | Place the full cylinder into the harness assembly. NOTE: If a second firefighter assists, the second firefighter shows/reports the pressure to the wearer. |  |  |
| 8. | Inspect the full cylinder’s valve opening and high-pressure hose fitting for debris. |  |  |
| 9. | Securely connect the high-pressure hose to the cylinder. |  |  |
| 10. | Slowly and fully open the cylinder valve and listen for an audible alarm and leaks as the system pressurizes. |  |  |
| 11. | Connect regulator and take normal breaths. |  |  |
| 12. | Check cylinder and remote pressure gauges. |  |  |