Skill Sheet 7-11

Objective 18	Hoist a roof ladder. [NFPA 4.1.2, 4.3.20]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will tie approved knots and hoist a roof ladder. This skill requires students to work with as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

CAUTION: Because ladders are unwieldy, they should be steadied with a tag line to keep them from bouncing against the side of the building while being raised.

Resources

| - Roof ladder - One 50-foot (15 m) length of utility rope | - Tag/guide line - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 7-11
Assessment

Objective 18	Hoist a roof ladder. [NFPA 4.1.2, 4.3.20]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Lower an appropriate length of rope from the intended destination of the ladder. |  |  |
| 2. | Make a large loop in the end of the rope using a figure-eight on a bight. |  |  |
| 3. | Place the closed loop under the ladder and bring it up between the rung about one-third the distance from the hoisting end. |  |  |
| 4. | Open the loop and place it over the tip of the ladder. |  |  |
| 5. | Arrange the standing part under the ladder rungs. |  |  |
| 6. | Tighten the loop around the beams, pulling the standing part of the rope up behind rungs toward ladder tip. |  |  |
| 7. | Tie a tag/guide line to the ladder. |  |  |
| 8. | Hoist the ladder. |  |  |