Skill Sheet 8-1

Objective 11	Clean, inspect, and maintain a ladder. [NFPA 1001, 4.5.1]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will clean, inspect, and maintain a ladder. The steps can be used for a variety of ladder types including straight, extension, roof, and pole ladders.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Ladder - Stiff-bristled brush - Garden hose - Bucket - Saw horses | - Manufacturer-recommended cleaners and lubricants - Dry cloths - Ladder log and chalk or grease pen - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 8-1 
Assessment

Objective 11	Clean, inspect, and maintain a ladder. [NFPA 1001, 4.5.1]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Clean |  |  |
| 1. | Place the ladder flat on the sawhorses. |  |  |
| 2. | Clean all parts of the ladder with a stiff-bristled brush and cleaning solution. Remove greasy residues with approved cleaners. |  |  |
| 3. | Rinse the ladder thoroughly with clean water. |  |  |
| 4. | Dry the ladder thoroughly with clean, dry cloths. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Inspect |  |  |
| 1. | Inspect each part of the ladder, noting any: a. Looseness b. Cracks c. Dents d. Unusual wear e. Bent rungs or beams f. Heat damage, deformities, or change in sensor label |  |  |
| 2. | Circle any defects with chalk or a grease pen. |  |  |
| 3. | Extension ladders: Inspect the halyard for: a. Fraying or kinking b. Snugness of cable when in bedded position |  |  |
| 4. | Extension, roof, and pole ladders: Inspect all moveable parts. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Maintain |  |  |
| 1. | Lubricate parts as needed, using a recommended lubricant. |  |  |
| 2. | Extension ladders: Replace halyard, if necessary. |  |  |
| 3. | Tag and remove ladder from service for any conditions that cannot be corrected with cleaning, inspection, and simple maintenance. Notify officer. |  |  |
| 4. | Record cleaning, inspection, and maintenance performed. |  |  |