Skill Sheet 9-6

Objective 14	Force entry through a window. [NFPA 1001, 4.3.4, 4.3.11]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will force entry through a window. This skill covers only typical window construction that has glass panes. Students should complete both listed methods. When forcing entry into a fire building, firefighters must wear SCBA. Therefore, it is recommended that students wear SCBA for this training exercise. Remind students that they should always “try before they pry”.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Window prop - Forcible entry tool | - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 9-6
Assessment

Objective 14	Force entry through a window. [NFPA 1001, 4.3.4, 4.3.11]

CAUTION: Firefighters must maintain communication and coordinate actions at all times during forcible entry. Use safe glass breaking techniques when forcing entry through widows.

NOTE: Always remember to “try before you pry.”

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Glass Pane Window |  |  |
| 1. | Size up the situation. a. Try the window first. b. Evaluate window construction. |  |  |
| 2. | Break the window glass. a. Single-paned windows: Start at the top of the pane. b. Multiple-paned windows: Start at the lowest pane of glass. CAUTION: To avoid losing control of the tool, do not use excessive force. Keep hands and the tool above the point of impact. |  |  |
| 3. | Use the tool to clean all broken glass out of the frame. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Double Hung Window |  |  |
| 1. | Size up the situation. a. Try the window first. b. Evaluate window construction. |  |  |
| 2. | Insert the blade of an axe or other prying tool under the center of the bottom sash in line with the locking mechanism. |  |  |
| 3. | Pry upward on the tool handle to force the lock. |  |  |
| 4. | Push the lower sash upward to open the window. |  |  |