Skill Sheet 12-2

Objective 6	Inspect, clean, and maintain a hose. [NFPA 1001, 4.5.2]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will inspect, clean, and maintain a hose. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Used hose - Chalk or non-permanent marker - Fire hose service log - Cleaning supplies or hose-washing machine | - Replacement gaskets of appropriate types and sizes - Out-of-service tag (if applicable) - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 12-2
Assessment

Objective 6	Inspect, clean, and maintain a hose. [NFPA 1001, 4.5.2]

NOTE: Always use caution when handling couplings. Do not throw, drag, or drop couplings on the ground. Damage to the couplings can result in a bad connection.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Inspect |  |  |
| 1. | Stretch hose to full length on flat, clean, dry surface. a. Ensure grease or other chemicals cannot come into contact with hose. b. Attempt to unroll hose rather than drag it. |  |  |
| 2. | Inspect one coupling and its component parts (threads, attachment, lugs, swivel, and/or gasket). |  |  |
| 3. | Place the coupling back on the surface. |  |  |
| 4. | Walk along the section of hose, visually inspecting surface for abrasions, burns, or other damage. |  |  |
| 5. | Circle any damaged spots with chalk or non-permanent marker. |  |  |
| 6. | Inspect the other coupling and its component parts (threads, attachment, lugs, swivel, and/or gasket). |  |  |
| 7. | Turn hose over to inspect the other side. a. Follow same procedure and inspect hose back to coupling. b. Pay particular attention to marked locations on the other side of the hose. |  |  |
| 8. | Note general inspection results and update fire service hose log as required by local SOPs. a. If hose is damaged or has other defects, tag with out-of-service tag and remove from service until repaired and tested. b. If free of damage, return it to the appropriate location. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Clean |  |  |
| 1. | Hand wash or machine wash the hose according to local SOPs. |  |  |
| 2. | Dry hose according to local SOPs. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Maintain — Replace a Hose Gasket |  |  |
| 1. | Remove old or damaged gasket and discard in the proper receptacle. |  |  |
| 2. | Place new gasket into the groove in the swivel in which it is meant to sit, smoothing as necessary to seat. |  |  |