Skill Sheet 12-4

Objective 8	Make a donut hose roll. [NFPA 1001, 4.5.2]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will make a donut hose roll. Students should complete at least one of the listed methods.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

NOTE: If using two-way couplings (Storz or quarter-turn), the steps should be modified for that type of hose. References to male or female ends can be replaced with action at either end of the hose.

Resources

| - Hose | - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 12-4 
Assessment

Objective 8	Make a donut hose roll. [NFPA 1001, 4.5.2]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Method One |  |  |
| 1. | Lay the hose straight and flat on a clean surface. |  |  |
| 2. | Start the roll from a point 5 or 6 feet (1.5 or 1.8 m) off center toward the male coupling. |  |  |
| 3. | Roll the hose toward the female end, leaving sufficient space at the center loop to insert a hand for carrying. |  |  |
| 4. | Extend the short length of hose at the female end over the male threads to protect them. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Method Two |  |  |
| 1. | Lay the hose straight and flat on a clean surface. |  |  |
| 2. | Grasp either coupling and carry it to the opposite end. The looped section should lie flat, straight, and without twists. |  |  |
| 3. | Stand at the looped section, facing the couplings. |  |  |
| 4. | Start the roll on the male coupling side about 2½ feet (750 mm) from the bend (1½ feet [450 mm] for 1½-inch [38 mm] hose). |  |  |
| 5. | Roll the hose toward the male coupling. NOTE: If the hose behind the roll becomes tight during the roll, pull the female side back a short distance to relieve the tension. |  |  |
| 6. | Lay the roll flat on the ground as the roll approaches the male coupling. |  |  |
| 7. | Draw the female coupling end around the male coupling to complete the roll. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Twin Donut Roll Method |  |  |
| 1. | Lay the hose straight and flat on a clean surface and form two parallel lines from the loop end to the couplings, with the couplings next to each other. |  |  |
| 2. | Start the roll by folding the loop end over and upon the two hose lengths. |  |  |
| 3. | Roll both lengths simultaneously toward the coupling ends to form a twin roll. |  |  |
| 4. | Insert a strap through the center of the roll for carrying purposes. |  |  |