Skill Sheet 12-6

Objective 10	Make the accordion hose load. [NFPA 1001, 4.5.2]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will make the accordion hose load. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

NOTE: Hose should be loaded in a manner that allows it to deploy without the need to flip the couplings so that the hose does not catch in the bed.

Resources

| - Hose - Hose bed | - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 12-6
Assessment

Objective 10	Make the accordion hose load. [NFPA 1001, 4.5.2]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Lay the first length of hose in the bed on edge against the partition. |  |  |
| 2. | Fold the hose at the front of the hose bed back on itself. |  |  |
| 3. | Lay the hose back to the rear of the hose bed parallel to the first length. |  |  |
| 4. | Fold the hose at the rear of the hose bed so that the bend is even with the rear edge of the bed. |  |  |
| 5. | Lay the hose back to the front of the hose bed. |  |  |
| 6. | Continue laying the hose in parallel folds across the bed to complete the first tier. a. Stagger the folds at the rear edge of the bed so that every other bend is approximately 2 inches (50 mm) shorter than the edge of the bed. b. This stagger may also be done at the front. |  |  |
| 7. | Angle the hose upward to start the second tier. |  |  |
| 8. | Make the first fold of the second tier directly over the last fold of the first tier at the rear of the bed. |  |  |
| 9. | Continue with the second and succeeding tiers in the same manner as the first, progressively laying the hose in folds across the hose bed. |  |  |
| 10. | Move to the opposite hose bed and load the hose in the same manner as the first side. |  |  |
| 11. | When the load is complete, connect the last coupling on top with the female coupling from the first side. |  |  |
| 12. | Lay the connected couplings on top of the hose load. |  |  |
| 13. | Pull out the slack so that the crossover loop lies tightly against the hose load. |  |  |