Skill Sheet 12-8

Objective 12	Make the triple layer hose load. [NFPA 1001, 4.5.2]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will make the triple layer hose load. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

NOTE: Hose should be loaded in a manner that allows it to deploy without the need to flip the couplings so that the hose does not catch in the bed.

Resources

| - Hose - Hose bed - Rope or hose strap (optional) | - Nozzle - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 12-8
Assessment

Objective 12	Make the triple layer hose load. [NFPA 1001, 4.5.2]

NOTE: Start the load with the sections of hose connected and the nozzle attached.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Connect the female coupling to the discharge outlet. |  |  |
| 2. | Extend the hose in a straight line on the ground pointing straight away from the hose bed (either directly behind the apparatus or to the side). |  |  |
| 3. | Pick up the hose at a point two-thirds the distance the nozzle, creating a fold. |  |  |
| 4. | Carry this fold back to the apparatus and place it on the ground, creating an S-shaped configuration in the hose. |  |  |
| 5. | Adjust the hose so that the nozzle is a short distance back from the fold (1 foot [300 mm]). |  |  |
| 6. | Using several firefighters, pick up the entire length of the three layers, one at a time starting with the piece attached to the discharge outlet. |  |  |
| 7. | Begin laying the hose into the bed by folding over the three layers into the hose bed. |  |  |
| 8. | Fold the layers over at the front of the bed. |  |  |
| 9. | Lay the layers back to the rear on top of the previously laid hose. a. If the hose compartment is wider than one hose width, alternate folds on each side of the bed. b. Make all folds at the rear even with the edge of the hose bed. |  |  |
| 10. | Continue to lay the hose into the bed in an S-shaped configuration until the entire length is loaded. |  |  |
| 11. | Optional: Secure the nozzle to the first set of loops using a rope or hose strap. |  |  |