Skill Sheet 13-1

Objective 6       Make soft-sleeve and hard-suction hydrant connections. [NFPA 1001, 4.3.15]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will make soft-sleeve and hard-suction hydrant connections. Students should complete both of the listed methods. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

NOTE: Students should practice coordinating their efforts during water supply operations.

Resources

| - Hard-suction and soft-sleeve hose - Connection adapter, if needed - Rubber mallet - Hydrant wrench | - Spanner wrench - Pumping apparatus - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 13-1
Assessment

Objective 6	Make soft-sleeve and hard-suction hydrant connections. [NFPA 1001, 4.3.15]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Soft-Sleeve Connection |  |  |
| 1. | Remove the hydrant cap. Use a spanner wrench if the cap is too tight. |  |  |
| 2. | Inspect the hydrant for exterior damage and check for debris or damage inside the outlet. |  |  |
| 3. | Place the hydrant wrench on the valve stem opening nut. |  |  |
| 4. | Flush the hydrant to ensure that it is free of debris. |  |  |
| 5. | Connect the intake hose to the pump intake. Hand tighten the connection. |  |  |
| 6. | Make the hydrant connection to the steamer outlet (use with adapter as needed). Hand tighten the connection. |  |  |
| 7. | Open the hydrant slowly until the hose is full. |  |  |
| 8. | Tighten any leaking connections using a rubber mallet or spanner wrench. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Hard-Suction Connection |  |  |
| 1. | Remove the hydrant cap. Use a spanner wrench if the cap is too tight. |  |  |
| 2. | Inspect the hydrant for exterior damage and check for debris or damage in the outlet. |  |  |
| 3. | Connect the intake hose to the hydrant or apparatus (depending on local SOPs). Hand tighten the connection. |  |  |
| 4. | Connect the opposite end to the hydrant or apparatus. Hand tighten the connection. |  |  |
| 5. | Tighten any leaking connections using a rubber mallet or spanner wrench. |  |  |