Skill Sheet 13-14

Objective 19	Advance a charged attack line up a ladder into a window. [NFPA 1001, 4.3.10]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will advance a charged attack line up a ladder into a window. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Charged 1½-inch (38 mm) or larger hoseline - Ladder raised to upper story window - Class I safety harness (if available) - Tool to sound the floor | - Rope hose tool or utility strap - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 13-14 
Assessment

Objective 19	Advance a charged attack line up a ladder into a window. [NFPA 1001, 4.3.10]

NOTE: Make sure the ladder is properly secured, either by another firefighter or by mechanical means. Firefighters should be positioned on the same side of the hose.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Nozzle firefighter: Climb the ladder, carrying the nozzle. |  |  |
| 2. | Nozzle firefighter: Lock in with leg lock or Class I safety harness, leaving hands free to control and advance the hose. |  |  |
| 3. | Firefighters below: Feed the hose to the nozzle firefighter. |  |  |
| 4. | Nozzle firefighter: Sound the floor for stability and check that no victims are in the way. |  |  |
| 5. | Nozzle firefighter: Lay the nozzle on the window, and then enter the window. |  |  |
| 6. | Firefighters below: Climb the ladder, maintaining appropriate distance from each other. |  |  |
| 7. | Firefighters on the ladder: Lock in with leg lock or Class I safety harness once backup firefighter is in position opposite the window. Leave hands free to control and advance the hose. |  |  |
| 8. | Backup firefighter: Enter the window. |  |  |
| 9. | Firefighters on the ladder: Feed the hose to the nozzle and backup firefighters until signaled to stop. |  |  |
| 10. | Firefighters on the ladder: Secure the hose to the ladder with a rope hose tool or utility strap. |  |  |