Skill Sheet 13-17

Objective 22	Operate a small hoseline using the one-firefighter method. [NFPA 1001, 4.3.10]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will operate a small hoseline using the one-firefighter method.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Charged 1½- or 1¾-inch (38 or 45 mm) hoseline - Rope hose tool or utility strap | - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 13-17 
Assessment

Objective 22	Operate a small hoseline using the one-firefighter method. [NFPA 1001, 4.3.10]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Position the hose so that it extends straight back for at least 10 feet (3 m). |  |  |
| 2. | Stand facing the objective with feet spread at least shoulder width apart. |  |  |
| 3. | Hold the hose with one hand directly behind the nozzle and the opposite hand on the nozzle shutoff valve. |  |  |
| 4. | Prepare to absorb the reaction force of the nozzle using one of the following methods: a. Anchor the hose by placing a foot on it. Make an “S” so that the hose at the top of the “S” rests against the hip or waist area. Place the back foot on the bottom of the “S.” b. Attach a hose strap or webbing to the hose and then place it over the shoulder. |  |  |
| 5. | Operate the nozzle. |  |  |