Skill Sheet 13-18

Objective 23	Operate a large hoseline for exposure protection using the one-firefighter method. [NFPA 1001, 4.3. 8]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will operate a large hoseline for exposure protection using the one-firefighter method.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Charged 2½- or 3-inch (65 or 77 mm) hoseline - Rope hose tool or utility strap | - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 13-18 
Assessment

Objective 23	Operate a large hoseline for exposure protection using the one-firefighter method. [NFPA 1001, 4.3.8]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Form a loop immediately behind the nozzle using approximately 25 feet (7.5 m) of the hose. |  |  |
| 2. | Pass the nozzle beneath the loop so that the loop rests on the end of the hose approximately 2 feet (600 mm) behind the nozzle. |  |  |
| 3. | Secure the loop by tying the hose at the crossover point with a hose strap. |  |  |
| 4. | Kneel or sit on the hose at the crossover point. |  |  |
| 5. | Hold the hose with one hand directly behind the nozzle and the opposite hand on the nozzle shutoff valve. |  |  |
| 6. | Operate the nozzle. |  |  |