Skill Sheet 14-10

Objective 17	Extinguish a fire in a trash container. [NFPA 1001, 4.3.8]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will extinguish a fire in a trash container Additional firefighters should be standing by with a backup hoseline A safety officer should check each student’s gear before the student proceeds with the training evolution. Before proceeding with live-fire training evolutions, read and adhere to NFPA 1403, Standard on Live Fire Training Evolutions. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Exterior Class A fire in a large commercial type trash container - 1½-inch (38 mm) or larger charged backup hoseline - 1½-inch (38 mm) or larger uncharged attack hoseline | - Pike pole and trash hook - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 14-10 
Assessment

Objective 17	Extinguish a fire in a trash container. [NFPA 1001, 4.3.8]

CAUTION: Always don appropriate PPE, including SCBA, when on the fireground. Approach from uphill and upwind if possible. Maintain communication and situational awareness. Observe fire conditions throughout the operation.

NOTE: Firefighters must confirm the order to attack the fire prior to performing suppression operations.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Deploy and advance an uncharged attack hoseline. |  |  |
| 2. | Signal the driver/operator when ready for water. |  |  |
| 3. | Open the nozzle to bleed air from the line and check for adequate water flow. |  |  |
| 4. | Select the correct nozzle pattern and close the nozzle. |  |  |
| 5. | Advance to the trash container. Keep the stream between the container and any exposures. |  |  |
| 6. | Cool the outside of the container and any exposures. |  |  |
| 7. | Extinguish the fire in the container. |  |  |
| 8. | Search for and extinguish hidden fires. a. Break up material and probe with a pike pole to search for hot spots. b. Extinguish hot spots. |  |  |