Skill Sheet 15-10

Objective 13	Construct a water chute and attach it to a catchall. [NFPA 1001, 4.3.14]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will construct a chute and attach it to a catchall. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Salvage cover - Catchall | - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 15-10 
Assessment

Objective 13	Construct a chute and attach it to a catchall. [NFPA 1001, 4.3.14]

NOTE: These steps are done with both firefighters performing the steps simultaneously.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Open the salvage cover. |  |  |
| 2. | Lay the cover flat at the desired location. |  |  |
| 3. | Roll the opposite edges of the salvage cover toward the center until there is 1 to 3 feet (300 to 900 mm) between the rolls. |  |  |
| 4. | Turn the cover over, keeping the rolls in place and flattening the center to the floor. |  |  |
| 5. | Slide the end of the chute under one corner of the catchall, about 1 to 2 feet (300 to 600 mm). |  |  |
| 6. | Unfold the corner of the catchall. |  |  |
| 7. | Flatten the corner of the catchall to form a seamless path for the water. |  |  |