Skill Sheet 15-2

Objective 5	Roll a salvage cover for a one-firefighter spread. [NFPA 1001, 4.3.14]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will roll a salvage cover for a one-firefighter spread. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Salvage cover - Inner tube bands, Velcro® straps, or tie cords | - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 15-2
Assessment

Objective 5	Roll a salvage cover for a one-firefighter spread. [NFPA 1001, 4.3.14]

NOTE: Two firefighters must make initial folds to reduce the width of the cover. Steps 1 through 8 are performed simultaneously by both firefighters on opposite sides of the cover. Steps 9 through 12 may be performed by both firefighters who are stationed at the same end of the roll.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Grasp the cover with the outside hand midway between the center and the edge to be folded. |  |  |
| 2. | Place the other hand on the cover as a pivot midway between the outside hand and the center. |  |  |
| 3. | Bring the fold over to the center of the cover, creating an inside fold (center) and an outside fold. |  |  |
| 4. | Grasp the corner with the outside hand. |  |  |
| 5. | Place the other hand as a pivot on the cover over the outside fold. |  |  |
| 6. | Bring this outside edge over to the center, and place it on top of and in line with the previously placed first fold. |  |  |
| 7. | Fold the other half of the cover in the same manner. |  |  |
| 8. | Straighten the folds. |  |  |
| 9. | Fold over about 12 inches (300 mm) at each end of the cover to make clean, even ends for the completed roll. |  |  |
| 10. | Start by rolling and compressing one end into a tight compact roll. Roll toward the opposite end. |  |  |
| 11. | Tuck in any wrinkles that form ahead of the roll as the roll progresses. |  |  |
| 12. | Secure the completed roll with inner tube bands or Velcro® straps or tie with cords. |  |  |