Skill Sheet 15-6

Objective 9	Fold a salvage cover for a two-firefighter spread. [NFPA 1001, 4.3.14]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will fold a salvage cover for a two-firefighter spread. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Salvage cover | - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 15-6
Assessment

Objective 9	Fold a salvage cover for a two-firefighter spread. [NFPA 1001, 4.3.14]

NOTE: Two firefighters must make initial folds to reduce the width of the cover. Steps 1 through 11 are performed simultaneously by both firefighters. Steps 12 through 18 are performed by the respective firefighters. Steps 19 through 21 are performed simultaneously by both firefighters.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | With the cover stretched lengthwise, grasp opposite ends of the cover at the center grommet. |  |  |
| 2. | Pull the cover tightly between each firefighter. |  |  |
| 3. | Raise the center fold high above the ground. |  |  |
| 4. | Shake out the wrinkles to form the first half-fold. |  |  |
| 5. | Spread the half-fold on the ground. |  |  |
| 6. | Smooth the half-fold flat to remove the wrinkles. |  |  |
| 7. | Stand at each end of the half-fold and face the cover. |  |  |
| 8. | Grasp the open-edge corners. |  |  |
| 9. | Place a foot at the center of the half-fold, making a pivot for the next fold. |  |  |
| 10. | Stretch the part of the cover being folded tightly between each firefighter. |  |  |
| 11. | Make the quarter-fold by folding the open edges over the folded edge. |  |  |
| 12. | Firefighter #1: Stand on one end of the quarter-fold. |  |  |
| 13. | Firefighter #2: Grasp the opposite end and shake out all the wrinkles. |  |  |
| 14. | Firefighter #2: Carry this end to the opposite end, maintaining alignment of outside edges. |  |  |
| 15. | Both Firefighters: Place the carried end on the opposite end, aligning all edges. |  |  |
| 16. | Both Firefighters: Place the folded cover on the ground and position at opposite ends. |  |  |
| 17. | Firefighter #2: Stand on the folded end of the cover. |  |  |
| 18. | Firefighter #1: Shake out all wrinkles and align all of the edges. |  |  |
| 19. | Grasp the open ends and use the inside foot as a pivot for the next fold. |  |  |
| 20. | Bring the open ends over and place them just short of the center fold. |  |  |
| 21. | Fold the opposite side in the same manner. |  |  |