Skill Sheet 17-4

Objective 11	Lift a wheel-resting passenger vehicle using a jack. [NFPA 1001, 5.4.1]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will lift a wheel-resting passenger vehicle using a jack. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Wheel-resting passenger vehicle - Wheel chocks - Cribbing | - Jack - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 17-4 
Assessment

Objective 11	Lift a wheel-resting passenger vehicle using a jack. [NFPA 1001, 5.4.1]

WARNING: Rescuers must ensure that the vehicle is properly stabilized and the scene is safe.

CAUTION: Rescuers must take necessary precautions to protect themselves and victims from hazards including, but not limited to: glass fragments and dust, jagged metal, SRS gas cylinders, undeployed airbags, and fire hazards.

NOTE: Monitor equipment throughout the operation and make adjustments as needed.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Identify vehicle’s construction, condition, and integrity. |  |  |
| 2. | Provide initial stabilization. |  |  |
| 3. | Identify a suitable lift point and support locations. |  |  |
| 4. | Verify that the surface under the support locations will support the weight of the vehicle and equipment. Construct a solid base or use alternative actions to provide base support, if necessary. |  |  |
| 5. | Select the type of jack to be used and position the jack so that it is directly beneath a suitable lift point. WARNING: Do not lie beneath the vehicle while positioning the jack, as it may result in serious injury or death if the vehicle is improperly stabilized. |  |  |
| 6. | Operate the jack until desired lift is achieved. |  |  |
| 7. | Capture progress throughout the lift. |  |  |
| 8. | Once lift is achieved, verify that progress has been captured and lower the jack until the vehicle is resting firmly on the cribbing. |  |  |
| 9. | Monitor and maintain the integrity of the cribbing. |  |  |