Skill Sheet 17-9

Objective 16	Displace a dashboard. [NFPA 1001, 5.4.1]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will displace a dashboard. Students must complete at least one of the listed methods. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Vehicle - Hydraulic cutters - Hydraulic spreaders | - Hydraulic ram - Cribbing - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 17-9
Assessment

Objective 16	Displace a dashboard. [NFPA 1001, 5.4.1]

WARNING: Rescuers must ensure that the vehicle is properly stabilized and the scene is safe.

CAUTION: Rescuers must take necessary precautions to protect themselves and victims from hazards including, but not limited to: glass fragments and dust, jagged metal, SRS gas cylinders, undeployed airbags, and fire hazards.

NOTE: Monitor equipment throughout the operation and make adjustments as needed.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| Jacking or Lifting with Spreaders |  |  |  |
| 1. | Remove the front door. |  |  |
| 2. | Make relief cuts behind the strut mounts to eliminate movement of the front end of the vehicle. |  |  |
| 3. | Cut the upper portion of the A-post if the roof is intact. |  |  |
| 4. | Create a purchase point in the lower portion of the A-post which is large enough to accommodate the spreader tips to the desired depth. Create the purchase point between the door hinges, if possible. |  |  |
| 5. | Place cribbing between the base of the A-post and the surface beneath. |  |  |
| 6. | Insert the spreader tips into the purchase point on the A-post. |  |  |
| 7. | Open the spreaders to lift the dash until sufficient clearance is achieved, while maintaining capture. |  |  |
| 8. | Monitor and maintain the integrity of the cribbing. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Pushing or Rolling a Dashboard |  |  |
| 1. | Remove the front door. |  |  |
| 2. | Make relief cuts behind the strut mounts to eliminate movement of the front end of the vehicle. |  |  |
| 3. | Cut the upper portion of the A-post if the roof is still intact. |  |  |
| 4. | Cut the bottom portion of the A-post, below the bottom door hinge, if possible. |  |  |
| 5. | Place cribbing between the rocker panel and the surface beneath. |  |  |
| 6. | Position the ram between the base of the B-post and on an area just above the top hinge on the A-post. |  |  |
| 7. | Extend the ram to move the dash until sufficient clearance is achieved. NOTE: Additional relief cuts may be needed during the operation. If tools need to be removed, a wedge can be placed within the void to prevent the return or lowering of the dash. |  |  |