Skill Sheet 18-1

Objective 5	Place a foam line in service — In-line eductor. [NFPA 1001, 5.3.1]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skills evaluation checklist, students will place a foam line in service. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Foam eductor - Hose and nozzle compatible with eductor - Foam concentrate or simulated concentrate | - Pumping apparatus - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 18-1
Evaluation Checklist

Objective 5	Place a foam line in service — In-line eductor. [NFPA 1001, 5.3.1]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Select the proper type and quantity of foam concentrate for the fuel involved. |  |  |
| 2. | Place the foam concentrate at the eductor. |  |  |
| 3. | Check the eductor and nozzle for hydraulic compatibility (rated for the same flow). |  |  |
| 4. | Adjust the eductor metering valve to the same percentage rating as that listed on the foam concentrate container. |  |  |
| 5. | Attach the eductor to a hose capable of efficiently flowing the rated capacity of the eductor and the nozzle. |  |  |
| 6. | Attach the hoseline and nozzle to the discharge end of the eductor. Ensure that there are no kinks in the hose. |  |  |
| 7. | Place the eductor suction hose into the foam concentrate. |  |  |
| 8. | Open the nozzle. |  |  |
| 9. | Increase the water supply pressure to that required for the eductor. Consult the manufacturer’s recommendations for the specific eductor. |  |  |