Skill Sheet 5-2

Objective 11	Don SCBA. [NFPA 4.3.1]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will don SCBA. Students must complete at least one of the two methods listed. The steps given in this skill sheet are general procedures for donning SCBA. Specific Other methods may vary by department according to local policy.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Structural fire fighting protective clothing | - SCBA including PASS device |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 5-2
Assessment

Objective 11	Don SCBA. [NFPA 4.3.1]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| Over-The-Head Method |  |  |  |
| 1. | Ensure the cylinder is full (at least 90% capacity). |  |  |
| 2. | Position the SCBA. Ensure that all harness straps are fully extended and untangled. |  |  |
| 3. | Open cylinder valve fully. Listen for the activation of the integrated PASS Alarm if equipped. Listen for the activation of the low air alarm. |  |  |
| 4. | Check cylinder and regulator pressure gauges. |  |  |
| 5. | Grab the back frame so that the shoulder straps will be outside of the arms. Using proper lifting technique, raise the SCBA overhead while guiding elbows into the loops formed by shoulder straps. |  |  |
| 6. | Release the harness assembly and allow the SCBA to slide down the back. |  |  |
| 7. | Fasten chest strap (if equipped), buckle waist strap, and adjust shoulder straps. |  |  |
| 8. | Don facepiece over the head and securely tighten the straps. |  |  |
| 9. | Test the facepiece for a proper seal and operation of the exhalation valve. NOTE: Not all facepieces are designed for a seal check without the regulator being attached and activated. |  |  |
| 10. | Don hood. |  |  |
| 11. | Don helmet. |  |  |
| 12. | Connect/activate air supply. |  |  |
| 13. | Activate external PASS device (if not equipped with integrated device). |  |  |
| 14. | Don gloves. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Coat Method |  |  |
| 1. | Ensure the cylinder is full (at least 90% capacity). |  |  |
| 2. | Position the SCBA. Ensure that all harness straps are fully extended and untangled. |  |  |
| 3. | Open cylinder valve fully. Listen for the activation of the integrated PASS Alarm if equipped. Listen for the activation of the low air alarm. |  |  |
| 4. | Check cylinder and regulator pressure gauges. |  |  |
| 5. | Grasp the shoulder straps on the SCBA and raise the SCBA. |  |  |
| 6. | Guide elbows through the loops one arm at a time and swing SCBA around shoulders, allowing SCBA to come to rest in the proper position. |  |  |
| 7. | Fasten chest strap (if equipped), buckle waist strap, and adjust shoulder straps. |  |  |
| 8. | Don facepiece over the head and securely tighten the straps. |  |  |
| 9. | Test the facepiece for a proper seal and operation of the exhalation valve. NOTE: Not all facepieces are designed for a seal check without the regulator being attached and activated. |  |  |
| 10. | Don hood. |  |  |
| 11. | Don helmet. |  |  |
| 12. | Connect/activate air supply. |  |  |
| 13. | Activate external PASS device (if not equipped with integrated device). |  |  |
| 14. | Don gloves. |  |  |