Skill Sheet 5-4

Objective 13	Doff personal protective equipment, including SCBA, and prepare for reuse. [NFPA 4.1.2, 4.3.3]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will doff personal protective equipment and SCBA and prepare it for reuse. Inform students of any time requirements for this skill.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Structural fire fighting protective clothing | - SCBA including PASS device |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 5-4
Assessment

Objective 14	Doff personal protective equipment, including SCBA, and prepare for reuse. [NFPA 4.1.2, 4.3.3]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Doff SCBA |  |  |
| 1. | Loosen straps and remove SCBA. |  |  |
| 2. | Close cylinder valve completely. |  |  |
| 3. | Bleed air from high- and low-pressure hoses. Listen for low air alarm activation. |  |  |
| 4. | Reset PASS device according to manufacturer specifications. |  |  |
| 5. | Check air cylinder pressure and replace or refill cylinder. |  |  |
| 6. | Return all straps, valves, and components back to ready state. |  |  |
| 7. | Inspect SCBA and facepiece for damage and need for cleaning. |  |  |
| 8. | Clean equipment as needed. Remove damaged equipment from service and report damage to company officer. |  |  |
| 9. | Place SCBA back in storage area so that it is ready for immediate use. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Doff PPE |  |  |
| 1. | Remove PPE. |  |  |
| 2. | Inspect PPE for damage. |  |  |
| 3. | Clean and dry equipment as needed. |  |  |
| 4. | Remove damaged equipment from service and report damage to company officer. |  |  |
| 5. | Place PPE in a ready state. |  |  |