Skill Sheet 13-10

Objective 15	Advance a hoseline into a structure. [NFPA 1001, 4.3.10]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will advance a hoseline into a structure. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

NOTE: Whenever possible, position firefighters at critical points (obstructions and corners) to help feed the hose and keep the hoseline free of kinks.

Resources

| - Hoseline - Pumping apparatus - Structure | - Thermal imager - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 13-10 
Assessment

Objective 15	Advance a hoseline into a structure. [NFPA 1001, 4.3.10]

NOTE: Firefighters should be positioned on the same side of the hose.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Unload the hose. |  |  |
| 2. | Face the nozzle with about 15 to 20 feet (5 to 6 m) of hose between each firefighter. |  |  |
| 3. | Place the hose over one shoulder. |  |  |
| 4. | Fully open SCBA before approaching the structure entrance or entering the smoke environment. |  |  |
| 5. | Advance the hose to the building entrance, but do not enter the building. a. Size up the environment to identify hazards. b. Approach the door from the side opposite the hinges. |  |  |
| 6. | Signal the driver/operator to charge the hoseline. |  |  |
| 7. | Open the nozzle fully to ensure adequate water flow and to allow the pump operator to set the pressure. |  |  |
| 8. | Set the desired nozzle pattern and bleed air from the hoseline. |  |  |
| 9. | Check for heat using a thermal imager. |  |  |
| 10. | Communicate readiness to enter the structure. |  |  |
| 11. | Enter the structure when directed to do so. Stay low and maintain spacing. |  |  |