Skill Sheet 13-19

Objective 24	Operate a large hoseline using the two-firefighter method. [NFPA 1001, 4.3.8, 4.3.10]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will operate a hoseline using the two-firefighter method. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Charged 2½- or 3-inch (65 or 77 mm) hoseline - Rope hose tool or utility strap (optional) | - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 13-19 
Assessment

Objective 24	Operate a large hoseline using the two-firefighter method. [NFPA 1001, 4.3.8, 4.3.10]

NOTE: Firefighters should be positioned on the same side of the hose.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Nozzle firefighter: Hold the hose with one hand directly behind the nozzle and the opposite hand on the shutoff valve. |  |  |
| 2. | Backup firefighter: Grasp the hose with both hands. |  |  |
| 3. | Nozzle firefighter: Operate the nozzle. |  |  |
| 4. | Backup firefighter: Absorb the reaction force of the nozzle and assist the nozzle firefighter in controlling the elevation of the stream. NOTE: Hose straps can be used to better control the hose, if necessary. If used, attach straps and loops so that each person shares the backward force from the nozzle. |  |  |