Skill Sheet 13-2

Objective 7      Connect and place a hard-suction hose for drafting from a static water source. [NFPA 1001, 4.3.15]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will connect and place a hard-suction hose for drafting from a static water source. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Floating or conventional barrel-type hose strainer - Spare hard-suction hose coupling gasket(s) - Utility rope or webbing - Rubber mallet | - Two 10-foot (3m) sections of hard-suction hose - Pumping apparatus - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 13-2 
Assessment

Objective 7	Connect and place a hard-suction hose for drafting from a static water source. [NFPA 1001, 4.3.15]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Check the hard-suction couplings. a. Remove any dirt or debris. b. Replace worn gaskets. |  |  |
| 2. | Connect the sections of hard-suction hose. a. Align sections. b. Hand-tighten the connection. c. Use rubber mallet to make an airtight connection, if necessary. d. Keep the hose off of the ground. |  |  |
| 3. | Connect the strainer to one end of the hard-suction hose. a. Hand-tighten the connection. b. Use a rubber mallet to make an airtight connection, if necessary. c. Fasten the rope or webbing to the strainer. |  |  |
| 4. | Put the strainer into the water. If using a barrel strainer, use the rope to maneuver the hose and keep the strainer off the bottom. |  |  |
| 5. | Prepare the pump intake for coupling by removing the cap and keystone intake valve, if applicable. |  |  |
| 6. | Connect the hard-suction hose to the pump intake. a. Align the sections. b. Hand-tighten the connection. |  |  |
| 7. | Tie the strainer rope (if used) to the pumper or stationary object. |  |  |
| 8. | Dismantle drafting equipment and return to proper storage per local SOPs. |  |  |