Skill Sheet 13-20

Objective 25	Deploy and operate a master stream device. [NFPA 1001, 4.3.8]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will deploy and operate a master stream device. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Two-way portable monitor unit or LDH-supplied monitor unit - Attack pumper | - Two lengths of 2½-inch (65 mm) or larger hose - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 13-20
Assessment

Objective 25	Deploy and operate a master stream device. [NFPA 1001, 4.3.8]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Remove the monitor unit from the apparatus and carry it to the set-up area. |  |  |
| 2. | Position the monitor unit on a solid, level surface. |  |  |
| 3. | Secure the monitor. |  |  |
| 4. | Adjust the nozzle to the proper elevation. |  |  |
| 5. | Secure the anchor lock, if applicable. |  |  |
| 6. | Extend the hoselines to the monitor unit. |  |  |
| 7. | Connect the hoselines to the monitor unit. |  |  |
| 8. | Hand tighten the swivel couplings. |  |  |
| 9. | Check the tip size, ensuring that you have the proper tip for the situation, or select the desired stream pattern. |  |  |
| 10. | Signal the driver/operator to charge the hoseline. |  |  |
| 11. | Steady the monitor. |  |  |
| 12. | Adjust the direction of water flow as necessary. |  |  |
| 13. | Operate the master stream device by aiming the stream in the correct direction to hit the designated target. |  |  |