Skill Sheet 13-4

Objective 9	Make a hydrant connection from a forward lay. [NFPA 1001, 4.3.15]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will make a hydrant connection from a forward lay. This skill requires students to work as a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Hose - Pumping apparatus | - Hydrant - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 13-4
Assessment

Objective 9	Make a hydrant connection from a forward lay. [NFPA 1001, 4.3.15]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Firefighter #1: Pull enough supply hose from the apparatus to reach and wrap around the hydrant. |  |  |
| 2. | Firefighter #1: Secure a loop of hose around the hydrant. Wrap the hose around the hydrant in a manner that restrains it when the pumper moves away from the hydrant. |  |  |
| 3. | Firefighter #1: Signal the driver/operator to proceed and deploy the hose to the incident. |  |  |
| 4. | Firefighter #1: Connect the supply hose to the hydrant. a. Remove the cap from the hydrant. b. Place the hydrant wrench on the valve stem operating nut. c. Flush the hydrant. d. Connect the hose to the appropriate outlet. |  |  |
| 5. | Firefighter #2: Complete the hose lay to the scene. |  |  |
| 6. | Firefighter #2: Connect the hose to the fire pump intake valve. |  |  |
| 7. | Firefighter #1: Slowly and fully open the hydrant. |  |  |
| 8. | Firefighter #1: Proceed along the hose to the pumper, removing kinks and checking for leaks. |  |  |