Skill Sheet 13-6

Objective 11	Advance a hose load. [NFPA 1001, 4.3.10]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will advance a hose load. Students should complete at least one of the listed methods. One method of this skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Preconnected hose load - Pumping apparatus | - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 13-6 
Assessment

Objective 11	Advance a hose load. [NFPA 1001, 4.3.15]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Flat Hose Method |  |  |
| 1. | Put one arm through the longer pull loop. |  |  |
| 2. | Grasp the shorter pull loop with the same hand. |  |  |
| 3. | Grasp the nozzle with the opposite hand. |  |  |
| 4. | Pull the load from the bed using the pull loops. |  |  |
| 5. | Advance toward the fire until the hose is fully extended, checking to be sure that the hose is free of kinks. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Minuteman Hose Method |  |  |
| 1. | Grasp the nozzle and pull loops, if provided. |  |  |
| 2. | Pull the load approximately one-third to one-half of the way out of the hose bed. |  |  |
| 3. | Face the direction of travel. |  |  |
| 4. | Place the hose load on the shoulder with the nozzle against your stomach. |  |  |
| 5. | Walk away from the apparatus, pulling the hose out of the bed by the pull loop. |  |  |
| 6. | Advance toward the fire until the hose is fully extended, allowing the load to play out from the top of the pile. Check to be sure the hose is free of kinks. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Triple Layer Method |  |  |
| 1. | Place the nozzle and the fold of the first tier over the shoulder. |  |  |
| 2. | Face the direction of travel. |  |  |
| 3. | Walk away from the apparatus. |  |  |
| 4. | Pull the hose completely out of the bed. |  |  |
| 5. | Drop the folded end from the shoulder when the hose bed has been cleared. |  |  |
| 6. | Advance toward the fire until the hose is fully extended, checking to be sure that the hose is free of kinks. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Shoulder Load Method |  |  |
| 1. | NOTE: Firefighters should be positioned on the same side of the hose. Firefighter #1: Attach the nozzle to the end of the hose, if desired. |  |  |
| 2. | Firefighter #2: Position at the tailboard facing the direction of travel. |  |  |
| 3. | Firefighter #2: Place the initial fold of hose over the shoulder so that the nozzle can be held across the front of the torso. |  |  |
| 4. | Firefighter #2: Bring the hose from behind back over the shoulders so that the rear fold ends at the back of the knee. |  |  |
| 5. | Firefighter #2: Make a fold in front that ends at knee height and bring the hose back over the shoulder. |  |  |
| 6. | Firefighter #2: Move forward approximately 15 feet (5 m). |  |  |
| 7. | Firefighter #3: Position at the tailboard facing the direction of travel. |  |  |
| 8. | Firefighter #3: Load hose onto the shoulder in the same manner as Firefighter #2 until an appropriate amount of hose is loaded. |  |  |
| 9. | Firefighter #1: Uncouple the hose from the hose bed and hand the coupling to the last firefighter. |  |  |
| 10. | Advance toward the fire until the hose is fully extended, checking to be sure that the hose is free of kinks. |  |  |