Skill Sheet 21-2

Objective 4	Inspect and maintain a portable generator and lighting equipment. [NFPA 1001, 5.5.4]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will inspect and maintain a portable generator and lighting equipment.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Maintenance tools as required by manufacturer - Cleaning rags - Spare or replacement parts such as spark plugs and lightbulbs | - Appropriate oils and fluids - Maintenance log and pen or pencil - Appropriate PPE including hearing, eye, and hand protection |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 21-2 
Assessment

Objective 4	Inspect and maintain a portable generator and lighting equipment. [NFPA 1001, 5.5.4]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Inspect and maintain spark plugs. a. Inspect for damage, visible corrosion, carbon accumulation, or cracks in the porcelain. b. Ensure that the spark plug wire is tight. c. Replace spark plugs if damaged or if the service manual recommends replacement. |  |  |
| 2. | Inspect the carburetor and identify signs of fuel leaks. |  |  |
| 3. | Check the fuel level and refill as needed. |  |  |
| 4. | Check the oil level and refill as needed. |  |  |
| 5. | Start the generator and run tests as required by the service manual. |  |  |
| 6. | Inspect and maintain lighting equipment. a. Inspect electrical cords for damaged insulation, exposed wiring, and missing or bent prongs. b. Connect each light to the generator one light at a time. c. Replace lightbulbs as necessary and discard faulty bulbs in an approved manner. |  |  |
| 7. | Record inspection and maintenance according to local SOPs. |  |  |