Skill Sheet 8-3

Objective 13	Carry a ladder using a two-firefighter carry. [NFPA 1001, 4.3.6, 4.3.11, 4.3.12]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will carry a ladder using the two-firefighter low-shoulder method or arm’s length on-edge method. Students should complete at least one of the listed methods. Remind students to use the proper technique to avoid back strain or injury when lifting a ladder from the ground. At the end of the skill, students may prepare to ground, raise, or stow the ladder. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

NOTE: A team leader should be designated before the skill begins. The leader ensures readiness and confirms all commands.

Resources

| - Single or extension ladder | - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 8-3 
Assessment

Objective 13	Carry a ladder using a two-firefighter carry. [NFPA 1001, 4.3.6, 4.3.11, 4.3.12]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Low-Shoulder Method |  |  |
| 1. | Visually inspect the work area. a. Inspect terrain for solid, level footing. b. Inspect overhead for electrical wires and obstructions. |  |  |
| 2. | Both firefighters: Kneel beside the ladder, facing the same direction. |  |  |
| 3. | Grasp the ladder beam. |  |  |
| 4. | Place the ladder on the beam. |  |  |
| 5. | Stand while shouldering the ladder. |  |  |
| 6. | Position the ladder for carrying. a. Lower the butt of the ladder slightly. b. Steady the ladder with both hands. |  |  |
| 7. | Carry the ladder forward toward the objective. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Arm’s Length On Edge Method |  |  |
| 1. | Visually inspect the work area. a. Inspect terrain for solid, level footing. b. Inspect overhead for electrical wires and obstructions. |  |  |
| 2. | Both firefighters: Kneel beside the ladder, facing the same direction. |  |  |
| 3. | Grasp the ladder beam. |  |  |
| 4. | Place the ladder on the beam. |  |  |
| 5. | Stand and lift the ladder to arm’s length. |  |  |
| 6. | Position the ladder for carrying. a. Grasp the beam. b. Place the ladder against your body. |  |  |
| 7. | Carry the ladder forward toward the objective. |  |  |