Skill Sheet 9-3

Objective 11	Force entry through an outward-swinging door. [NFPA 1001, 4.3.4]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will force entry through an outward-swinging door. Students should complete at least one of the listed methods. Although various forcible entry tools can be used to force doors, this skill sheet uses the Halligan tool. When forcing entry into a fire building, firefighters must wear SCBA. Therefore, it is recommended that students wear SCBA for this training exercise. Remind students that they should always “try before they pry.” This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Outward-swinging door prop - Flat-head axe | - Halligan tool - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 9-3 
Assessment

Objective 11	Force entry through an outward-swinging door. [NFPA 1001, 4.3.4]

CAUTION: Firefighters must maintain communication and coordinate actions at all times during forcible entry. Maintain control of the door at all times.

NOTE: Always remember to “try before you pry.”

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Removing the Hinges Method |  |  |
| 1. | Size up the door and lock. |  |  |
| 2. | Place the fork end of the Halligan between the top hinge and the door. |  |  |
| 3. | Pry up or down. |  |  |
| 4. | Twist the Halligan from side to side to loosen the hinge mounting screws, if necessary. |  |  |
| 5. | Pull the hinge clear of the door. |  |  |
| 6. | Repeat to pry and remove the other hinges. |  |  |
| 7. | Open the door. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Adz-end Method |  |  |
| 1. | Size up the door and lock. |  |  |
| 2. | Firefighter #1: Place the adz end of the Halligan just above or below the lock. If there are two locks, place the adz between the locks. |  |  |
| 3. | Firefighter #2: Strike the Halligan using the flat-head axe on the surface behind the adz, driving the adz into the space between the door and the jamb and past the interior door jamb. |  |  |
| 4. | Firefighter #1: Pry down and out, applying force to the forked end of the tool to separate the door from the jamb. |  |  |
| 5. | Open the door. |  |  |