Skill Sheet 9-9

Objective 17	Breach a metal wall with a rotary saw. [NFPA 1001, 4.3.4]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will force entry through a metal wall with a rotary saw. When forcing entry into a fire building, firefighters must wear SCBA. Therefore, it is recommended that students wear SCBA for this training exercise.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Wall prop - Rotary saw with metal cutting blade | - Hand tool - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 9-9 
Assessment

Objective 17	Force entry through a metal wall with a rotary saw. [NFPA 1001, 4.3.4]

CAUTION: Firefighters must maintain communication and coordinate actions at all times during forcible entry. Confirm that the utilities are off prior to forcing entry.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Size up the situation. a. Confirm that no other existing entry points are available. b. Evaluate wall construction. c. Consider the location of utilities. |  |  |
| 2. | Cut an inspection hole (small triangle) and utilize it to ensure that the area is safe to continue forcing entry. |  |  |
| 3. | Locate wall studs (indicated by wall screws). |  |  |
| 4. | Make a cut near the studs large enough for entry. Studs may be removed, if necessary. |  |  |
| 5. | Use a hand tool to remove wall material and insulation and place it out of the traffic area. |  |  |
| 6. | Use a hand tool to push inward and remove interior wall covering. |  |  |