Skill Sheet 10-11

Objective 18	Perform reduced profile maneuvers without removal of SCBA using the SCBA-first technique. [NFPA 1001 4.3.1, 4.3.5, 4.3.9]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will perform reduced profile maneuvers without removal of SCBA using the SCBA-first technique. The SCBA-first technique can be used effectively on interior stud spaces.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Restricted passage - Handheld portable radio - Handlight or flashlight | - Forcible entry tools - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 10-11
Assessment

Objective 18	Perform reduced profile maneuvers without removal of SCBA using the SCBA-first technique. [NFPA 1001 4.3.1, 4.3.5, 4.3.9]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Sit with SCBA and your back toward the opening. |  |  |
| 2. | Place one arm and the SCBA cylinder into the opening. |  |  |
| 3. | Using a backstroke technique, swim the other arm through the opening. |  |  |
| 4. | Using both arms and the wall board for leverage, pull through the space. |  |  |