Skill Sheet 10-12

Objective 19	Disentangle from debris or wires. [NFPA 1001 4.3.5]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will disentangle from debris or wires.

Always follow local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Materials to simulate entanglement - Handheld portable radio - Handlight or flashlight | - Forcible entry tools - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 10-12
Assessment

Objective 19	Disentangle from debris or wires. [NFPA 1001 4.3.5]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Recognize the emergency. Activate MAYDAY procedures as appropriate. |  |  |
| 2. | Decide the best steps to take to mitigate the situation and conserve energy. |  |  |
| 3. | If possible, back out of the entanglement or move forward with your SCBA pack down and the regulator protected with a gloved hand. |  |  |
| 4. | If unable to move backward or forward, attempt a reduced profile technique to locate points of entanglement. |  |  |
| 5. | Use a cutting tool to cut out of the entanglement, if possible. WARNING: Avoid cutting energized wires. If cutting is necessary, the cutting tool MUST be sufficiently insulated to withstand the electrical charge. |  |  |
| 6. | Consider a partial SCBA removal to assist in locating and removing points of entanglement. |  |  |
| 7. | If unable to escape, stay calm, communicate with the crew, and conserve air until the rescue crew arrives. |  |  |