Skill Sheet 14-1

Objective 8	Attack an interior structure fire at ground level using a direct, indirect, and combination attack. [NFPA 1001, 4.3.10]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will attack an interior structure fire using direct, indirect, and combination attack methods. Students must complete all three methods. Additional firefighters should be standing by with a backup hoseline. A safety officer should check each student’s gear before the student proceeds with the training evolution. Before proceeding with live-fire training evolutions, read and adhere to NFPA 1403, Standard on Live Fire Training Evolutions. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

NOTE: Remind students to advance based upon the environment. Depending on the situation, it may be necessary to crouch, crawl, or walk.

Resources

| - Class A live-fire building - Pumping apparatus - 1½-inch (38 mm) or larger charged backup hoseline - 1½-inch (38 mm) or larger uncharged attack hoseline | - Portable radios - Accountability system - Salvage and overhaul tools - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 14-1 
Assessment

Objective 8	Attack a structure fire at ground level using a direct, indirect, and combination attack. [NFPA 1001, 4.3.10]

CAUTION: Always don appropriate PPE, including SCBA, when on the fireground. Approach from uphill and upwind if possible. Maintain communication and situational awareness. Observe fire conditions throughout the operation.

NOTE: Firefighters must confirm the order to attack the fire prior to performing suppression operations.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Direct Attack Method |  |  |
| 1. | Deploy and advance an uncharged attack hoseline to a safe location near the point of entry. |  |  |
| 2. | Signal the driver/operator when ready for water. |  |  |
| 3. | Open the nozzle to bleed air from the line and check for adequate water flow. |  |  |
| 4. | Select the correct nozzle pattern and close the nozzle. |  |  |
| 5. | Enter the structure and advance to the seat of the fire. a. Extinguishing any fires that are encountered along the way. b. Cool hot gases overhead as needed using short applications of a solid or straight stream. CAUTION: Always maintain door control to control the flow path. |  |  |
| 6. | When in place near the seat of the fire, direct a solid or straight stream of water onto the base of the fire. |  |  |
| 7. | Locate and suppress any interior wall and subfloor fires as directed. |  |  |
| 8. | Close the nozzle when the fire is extinguished. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Indirect Attack Method |  |  |
| 1. | Deploy and advance an uncharged attack hoseline to a safe location near the point of entry. |  |  |
| 2. | Signal the driver/operator when ready for water. |  |  |
| 3. | Open the nozzle to bleed air from the line and check for adequate water flow. |  |  |
| 4. | Select the correct nozzle pattern and close the nozzle. |  |  |
| 5. | Enter the structure and advance to a location outside the fire compartment but as near to the seat of the fire as possible. a. Extinguish any fires that are encountered along the way. b. Cool hot gases overhead as needed using short applications of a solid or straight stream. CAUTION: Firefighters should not be inside the fire compartment while using the Indirect Attack Method. Always maintain door control to control the flow path. |  |  |
| 6. | When in place outside the fire compartment, open the nozzle and direct a fog pattern toward the ceiling and upper area of the walls. |  |  |
| 7. | Close the interior door to the compartment, allowing steam to develop. Crack the door to observe the conditions. |  |  |
| 8. | If necessary, open the door and continue to apply water to the compartment linings (walls and ceiling) until the fire is extinguished. |  |  |
| 9. | Close the nozzle when the fire is extinguished. |  |  |

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
|  | Combination Attack Method |  |  |
| 1. | Deploy and advance an uncharged attack hoseline to a safe location near the point of entry. |  |  |
| 2. | Signal the driver/operator when ready for water. |  |  |
| 3. | Open the nozzle to bleed air from the line and check for adequate water flow. |  |  |
| 4. | Select the correct nozzle pattern and close the nozzle. |  |  |
| 5. | Enter the structure and advance to the seat of the fire, extinguishing any fires that are encountered along the way. a. Extinguish any fires that are encountered along the way. b. Cool hot gases overhead as needed using short applications of a solid or straight stream. CAUTION: Always maintain door control to control the flow path. |  |  |
| 6. | When in place near the seat of the fire, open the nozzle and direct a narrow fog pattern toward the upper edge of the ceiling level. |  |  |
| 7. | Apply water using a T, Z, or O pattern, moving the stream from high to low, ensuring that the ceiling and floor are reached by the hose stream. |  |  |
| 8. | Close the nozzle when the room begins to darken. |  |  |
| 9. | Apply water using the direct attack method as needed. |  |  |
| 10. | Close the nozzle when the fire is extinguished. |  |  |