Skill Sheet 14-7

Objective 14	Attack a passenger vehicle fire. [NFPA 1001, 4.3.7]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will attack a passenger vehicle fire. Additional firefighters should be standing by with a backup hoseline. A safety officer should check each student’s gear before the student proceeds with the training evolution. Before proceeding with live-fire training evolutions, read and adhere to NFPA 1403, Standard on Live Fire Training Evolutions.  This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

NOTE: Piercing nozzles may be used to extinguish vehicle fires in some jurisdictions. The steps for this skilled may be altered as necessary if a piercing nozzle is used.

Resources

| - Training prop or simulated passenger vehicle fire - 1½-inch (38 mm) or larger charged backup line - 1½-inch (38 mm) or larger uncharged attack line - Pumping apparatus | - Halligan tool - Wheel chocks - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 14-7 
Assessment

Objective 14	Attack a passenger vehicle fire. [NFPA 1001, 4.3.7]

CAUTION: Always don appropriate PPE, including SCBA, when on the fireground. Approach from uphill, upwind, and at a 45-degree angle from the side of the vehicle, if possible. Maintain communication and situational awareness. Observe fire conditions throughout the operation. Ensure that the vehicle is properly stabilized.

NOTE: Firefighters must confirm the order to attack the fire prior to performing suppression operations.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Identify automobile fuel type, if possible. |  |  |
| 2. | Deploy an uncharged attack line. |  |  |
| 3. | Signal the driver/operator when ready for water. |  |  |
| 4. | Open the nozzle to bleed air from the line and check for adequate water flow. |  |  |
| 5. | Select the correct nozzle pattern. |  |  |
| 6. | Advance the attack line to the vehicle, applying water while advancing. |  |  |
| 7. | Extinguish any fire in the line of approach or under the vehicle. |  |  |
| 8. | Extinguish fire in the passenger compartment. |  |  |
| 9. | Open the hood and extinguish the fire in the engine compartment. NOTE: It may be necessary to force entry to the engine compartment. |  |  |
| 10. | Open the trunk and extinguish fire in the trunk. NOTE: It may be necessary to force entry to the trunk. |  |  |
| 11. | Extinguish hidden and smoldering fires. |  |  |
| 12. | Assess and control fuel leaks. |  |  |