Skill Sheet 14-8

Objective 15	Attack a fire in exterior stacked or piled Class A materials. [NFPA 1001, 4.3.8]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will attack a fire in exterior stacked or piled Class A materials. Additional firefighters should be standing by with a backup hoseline. A safety officer should check each student’s gear before the student proceeds with the training evolution. Before proceeding with live-fire training evolutions, read and adhere to NFPA 1403, Standard on Live Fire Training Evolutions. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Training prop or simulated stack fire - 1½-inch (38 mm) or larger charged backup hoseline - 1½-inch (38 mm) or larger uncharged attack hoseline | - Pumping apparatus - Pike pole or trash hook - Appropriate PPE including SCBA |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 14-8 
Assessment

Objective 15	Attack a fire in exterior stacked or piled Class A materials. [NFPA 1001, 4.3.8]

CAUTION: Always don appropriate PPE, including SCBA, when on the fireground. Approach from uphill and upwind if possible. Maintain communication and situational awareness. Observe fire conditions throughout the operation.

NOTE: Firefighters must confirm the order to attack the fire prior to performing suppression operations.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Deploy and advance an uncharged attack hoseline. |  |  |
| 2. | Signal the driver/operator when ready for water. |  |  |
| 3. | Open the nozzle to bleed air from the line and check for adequate water flow. |  |  |
| 4. | Select the correct nozzle pattern and close the nozzle. |  |  |
| 5. | Check for threat to exposures and cool exposures as necessary. |  |  |
| 6. | Position to make fire attack. |  |  |
| 7. | Extinguish the fire with a straight stream. |  |  |
| 8. | Expose fire in the debris using a pike pole or trash hook and extinguish any debris fires. |  |  |