Skill Sheet 7-12

Objective 19	Hoist a dry hoseline. [NFPA 4.1.2, 4.3.20]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will hoist a dry hoseline. This skill requires students to work with as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

CAUTION: Because hoselines are unwieldy, they should be steadied with a tag line to keep them from bouncing against the side of the building while being raised.

Resources

| - One 50-foot (15 m) length of utility rope - Uncharged hose with a nozzle | - Edge protection - Appropriate PPE |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 7-12
Assessment

Objective 19	Hoist a dry hoseline. [NFPA 4.1.2, 4.3.20]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Lower an appropriate length of rope from the intended destination of the hoseline. |  |  |
| 2. | Fold the nozzle end of the hoseline back over the rest of the hose so that an overlap of 4 to 5 feet (1.2 to 1.5 m) is formed. |  |  |
| 3. | Tie a clove hitch, with an overhand safety knot, around the tip of the nozzle and the hose it is folded against so that they are lashed together. |  |  |
| 4. | Place a half-hitch on the doubled hose about 12 inches (300 mm) from the loop end. |  |  |
| 5. | Hoist hoseline. |  |  |