Skill Sheet 17-6

Objective 13	Remove tempered vehicle glass. [NFPA 1001, 5.4.1]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will remove tempered vehicle glass. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Vehicle with tempered glass - Center punch or other tool | - Appropriate PPE - Eye and respiratory protection for victims |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 17-6
Assessment

Objective 13	Remove tempered vehicle glass. [NFPA 1001, 5.4.1]

WARNING: Rescuers must ensure that the vehicle is properly stabilized and the scene is safe.

CAUTION: Rescuers must take necessary precautions to protect themselves and victims from hazards including, but not limited to: glass fragments and dust, jagged metal, SRS gas cylinders, undeployed airbags, and fire hazards.

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Place a center punch or other tool in the lower corner of the window. |  |  |
| 2. | Brace the hand holding the center punch with the opposite hand to prevent it from pushing through the glass. |  |  |
| 3. | Break the window. |  |  |
| 4. | Use a tool to clear the remaining glass outward and away from the victim, if possible. CAUTION: Do not use hands to clear glass from the window. |  |  |