Skill Sheet 22-4

Objective #8	Prepare a preincident planning survey. [NFPA 1001, 5.5.1, 5.5.3]

| ## Student Name: |  | ## Date: |  |
| --- | --- | --- | --- |

Directions

For this skill sheet, students will prepare a preincident survey. This skill requires students to work as members of a team. Before the skill begins, indicate to students what position to take. Students should rotate positions to practice the skill as needed.

Always follow manufacturer’s recommendations and local standard operating procedures (SOPs) when performing all skills. Students must properly wear appropriate PPE when performing this skill.

Resources

| - Structure to survey - Copy of fire code and inspection manuals - Hard hat - Coveralls - Steel-toed shoes - Eye protection - Gloves | - 50-foot (15 m) tape measure - Flashlight - Camera - Inspection forms - Clipboard, pencils, paper, and preincident planning form |
| --- | --- |

Criteria & Evaluation Comments

Criteria (determined by the AHJ)

After the student has completed the skill sheet, write comments below.

Evaluator/Student Comments

| Pass |  |  | Fail |  |
| --- | --- | --- | --- | --- |

| Evaluator Signature | Date |  | Student Signature | Date |
| --- | --- | --- | --- | --- |

Skill Sheet 22-4 
Assessment

Objective 8	Prepare a preincident planning survey. [NFPA 1001, 5.5.1, 5.5.3]

| Task Steps |  | Yes | No |
| --- | --- | --- | --- |
| 1. | Contact the business owner or manager to gain permission to conduct the survey. a. Verify the correct address. b. Verify emergency contact information. |  |  |
| 2. | Record initial observations of the exterior of the structure. a. Number and location of fire hydrants, fire department connections, fire alarm boxes, rapid entry key systems, etc. b. Type of building construction and materials c. Types of exposures d. Access and egress from the site e. Building occupancy f. Construction or environmental features that could negatively impact fire suppression |  |  |
| 3. | Prepare a sketch of the building, streets, hydrants, etc. |  |  |
| 4. | Survey the interior of the structure, beginning on the lowest floor or the roof and record any features or conditions related to life safety and fire suppression. a. Location of fire protection systems, alarm panel, control valves, standpipes, etc. b. Location of exit stairwells, corridors, doors, etc. c. Hazardous operations, equipment, or materials d. Electrical control panels e. Life safety risks f. Roof access g. Potential ventilation openings h. Elevators i. High value contents or merchandise j. Potential fuel loads k. Any other potential hazards present |  |  |
| 5. | Draw a floor plan of the building that includes the information from Step 4. |  |  |
| 6. | Distribute the complete preincident plan according to local SOPs. |  |  |